| The relationship between SWR as seen at the transmitter end and the SWR at the antenna or load end is shown for a number of popular coax cable types. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values of measured SWR of 1.5, 2, 3 and 5:1 are used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The total loss for the coax is also shown, including both the matched loss and the additional loss due to the mismatch. For other lengths, the charts may be cascaded. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The data is taken using Transmission Line for Windows (TLW) as provided in recent additions of The ARRL Antenna Book. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Joel R. Hallas, W1ZR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 FT RG-58A (Belden 8259) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BAND | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  |
|  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  |
| 3.5 | 1.5 | 1.5 | 0.2 |  | 2 | 2.1 | 0.3 |  | 3 | 3.2 | 0.2 |  | 5 | 5.7 | 0.3 |  |
| 7 | 1.5 | 1.6 | 0.4 |  | 2 | 2.1 | 0.4 |  | 3 | 3.3 | 0.6 |  | 5 | 6.1 | 1.0 |  |
| 14 | 1.5 | 1.6 | 0.5 |  | 2 | 2.2 | 0.6 |  | 3 | 3.5 | 0.8 |  | 5 | 6.8 | 1.3 |  |
| 21 | 1.5 | 1.6 | 0.7 |  | 2 | 2.2 | 0.8 |  | 3 | 3.7 | 1.1 |  | 5 | 7.5 | 2.0 |  |
| 28 | 1.5 | 1.6 | 0.8 |  | 2 | 2.3 | 0.9 |  | 3 | 3.9 | 1.2 |  | 5 | 8.2 | 2.1 |  |
| 50 | 1.5 | 1.7 | 1.1 |  | 2 | 2.4 | 1.4 |  | 3 | 4.4 | 2.0 |  | 5 | 11.2 | 3.8 |  |
| 146 | 1.5 | 1.9 | 2.1 |  | 2 | 3.1 | 2.6 |  | 3 | 7.4 | 4.3 |  | 5 |  |  |  |
| 25 FT RG-8X (Belden 9258) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BAND | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  |
|  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  |
| 3.5 | 1.5 | 1.5 | 0.2 |  | 2 | 2.0 | 0.2 |  | 3 | 3.1 | 0.2 |  | 5 | 5.4 | 0.2 |  |
| 7 | 1.5 | 1.5 | 0.2 |  | 2 | 2.1 | 0.3 |  | 3 | 3.0 | 0.3 |  | 5 | 5.6 | 0.5 |  |
| 14 | 1.5 | 1.5 | 0.4 |  | 2 | 2.1 | 0.4 |  | 3 | 3.3 | 0.6 |  | 5 | 6.0 | 1.0 |  |
| 21 | 1.5 | 1.6 | 0.4 |  | 2 | 2.1 | 0.5 |  | 3 | 3.4 | 0.6 |  | 5 | 6.4 | 1.0 |  |
| 28 | 1.5 | 1.6 | 0.5 |  | 2 | 2.2 | 0.6 |  | 3 | 3.5 | 0.9 |  | 5 | 6.7 | 1.5 |  |
| 50 | 1.5 | 1.6 | 0.7 |  | 2 | 2.3 | 0.8 |  | 3 | 3.7 | 1.1 |  | 5 | 7.6 | 1.9 |  |
| 146 | 1.5 | 1.7 | 1.3 |  | 2 | 2.6 | 1.6 |  | 3 | 4.8 | 2.4 |  | 5 | 15.2 | 4.3 |  |
| 25 FT RG-213 (Belden 8267) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BAND | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  |
|  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  |
| 3.5 | 1.5 | 1.5 | 0.1 |  | 2 | 2.0 | 0.1 |  | 3 | 3.0 | 0.1 |  | 5 | 5.0 | 0.1 |  |
| 7 | 1.5 | 1.5 | 0.1 |  | 2 | 2.0 | 0.1 |  | 3 | 3.0 | 0.1 |  | 5 | 5.0 | 0.2 |  |
| 14 | 1.5 | 1.5 | 0.1 |  | 2 | 2.0 | 0.1 |  | 3 | 3.0 | 0.1 |  | 5 | 5.0 | 0.2 |  |
| 21 | 1.5 | 1.5 | 0.1 |  | 2 | 2.0 | 0.1 |  | 3 | 3.0 | 0.2 |  | 5 | 5.1 | 0.3 |  |
| 28 | 1.5 | 1.5 | 0.1 |  | 2 | 2.0 | 0.1 |  | 3 | 3.0 | 0.2 |  | 5 | 5.1 | 0.3 |  |
| 50 | 1.5 | 1.5 | 0.2 |  | 2 | 2.0 | 0.2 |  | 3 | 3.0 | 0.3 |  | 5 | 5.3 | 0.5 |  |
| 146 | 1.5 | 1.5 | 0.3 |  | 2 | 2.0 | 0.4 |  | 3 | 3.1 | 0.5 |  | 5 | 5.7 | 0.8 |  |
| 100 FT RG-58A (Belden 8259) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BAND | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  |
|  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  |
| 3.5 | 1.5 | 1.6 | 0.9 |  | 2 | 2.4 | 1.0 |  | 3 | 4.1 | 1.5 |  | 5 | 9.5 | 2.6 |  |
| 7 | 1.5 | 1.7 | 1.4 |  | 2 | 2.6 | 1.6 |  | 3 | 5.0 | 2.4 |  | 5 | 17.2 | 4.9 |  |
| 14 | 1.5 | 1.9 | 2.1 |  | 2 | 3.1 | 2.6 |  | 3 | 7.7 | 4.3 |  | 5 | INF | 66.4 |  |
| 21 | 1.5 | 2.1 | 2.7 |  | 2 | 3.7 | 3.6 |  | 3 | 13.7 | 7.0 |  | 5 | INF | 66.8 |  |
| 28 | 1.5 | 2.2 | 3.4 |  | 2 | 4.5 | 4.6 |  | 3 | 43.0 | 13.0 |  | 5 | INF | 67.3 |  |
| 50 | 1.5 | 3.0 | 4.9 |  | 2 | 10.4 | 8.2 |  | 3 | INF | 69.7 |  | 5 | INF | 69.7 |  |
| 146 | 1.5 | INF | 74.0 |  | 2 | INF | 74.0 |  |  |  |  |  |  |  |  |  |
| 100 FT RG-8X (Belden 9258) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BAND | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  |
|  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  |
| 3.5 | 1.5 | 1.6 | 0.6 |  | 2 | 2.2 | 0.8 |  | 3 | 3.6 | 1.1 |  | 5 | 7.2 | 1.9 |  |
| 7 | 1.5 | 1.6 | 1.0 |  | 2 | 2.3 | 1.2 |  | 3 | 4.1 | 1.7 |  | 5 | 9.3 | 3.2 |  |
| 14 | 1.5 | 1.7 | 1.4 |  | 2 | 2.6 | 1.7 |  | 3 | 4.9 | 2.6 |  | 5 | 16.5 | 5.5 |  |
| 21 | 1.5 | 1.8 | 1.7 |  | 2 | 2.8 | 2.1 |  | 3 | 6.0 | 3.3 |  | 5 | 43.4 | 8.8 |  |
| 28 | 1.5 | 1.9 | 2.1 |  | 2 | 3.1 | 2.8 |  | 3 | 7.5 | 4.3 |  | 5 | INF | 66.3 |  |
| 50 | 1.5 | 2.1 | 3.0 |  | 2 | 4.0 | 4.1 |  | 3 | 19.7 | 9.0 |  | 5 | INF | 67.0 |  |
| 146 | 1.5 | 3.9 | 6.5 |  | 2 | 678.0 | 23.5 |  | 3 | INF | 70.5 |  | 5 | INF | 69.2 |  |
| 100 FT RG-213 (Belden 8267) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BAND | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  | SWR | SWR | TOTAL |  |
|  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  | XMTR | LOAD | LOSS (dB) |  |
| 3.5 | 1.5 | 1.6 | 0.4 |  | 2 | 2.1 | 0.4 |  | 3 | 3.4 | 0.6 |  | 5 | 6.3 | 1.0 |  |
| 7 | 1.5 | 1.6 | 0.6 |  | 2 | 2.2 | 0.7 |  | 3 | 3.6 | 0.9 |  | 5 | 7.1 | 1.5 |  |
| 14 | 1.5 | 1.6 | 0.8 |  | 2 | 2.3 | 1.0 |  | 3 | 4.0 | 1.4 |  | 5 | 8.9 | 2.5 |  |
| 21 | 1.5 | 1.7 | 1.1 |  | 2 | 2.4 | 1.3 |  | 3 | 4.3 | 1.9 |  | 5 | 11.1 | 3.6 |  |
| 28 | 1.5 | 1.7 | 1.3 |  | 2 | 2.5 | 1.6 |  | 3 | 4.7 | 2.3 |  | 5 | 14.1 | 4.8 |  |
| 50 | 1.5 | 1.8 | 1.8 |  | 2 | 2.8 | 2.2 |  | 3 | 6.1 | 3.4 |  | 5 | 45.6 | 9.6 |  |
| 146 | 1.5 | 2.3 | 3.3 |  | 2 | 4.6 | 4.6 |  | 3 | 49.0 | 12.6 |  | 5 | INF | 67.3 |  |