AX25 driver for BAYCOM-like modem. Version of January 4, 1992

Free license for this software is granted for _amateur_ use only. Commercial usage is prohibited.

AX25.COM is a packet driver comforming (to some extend) to well known "FTP packet driver specification". It's purpose is to serve as interface between application software (e.g. KA9Q NOS) and a modem connected to RS232 port (e.g. BAYCOM modem).

The driver relies on the following RS232 pins:

RTS - controls PTT: high level activates the transmitter

DTR - transmitted data. On this pin the driver sends data to be transmitted by the modem.  When in receive state this pin is held high.

CTS - received data. The modem should supply here the data it receives.

DCD - the modem may supply here "carrier detect" signal. This pin is optional - it is not a must because the driver is able to deliver carrier signal from received data.

GND - a common ground for the above signals.

TxD outputs a signal which is in high (positive voltage) most of the time. A low power modem (TCM3105) can be supplied from RTS+DTR+TxD (adding realized with diodes).

My primary intention while writting this driver was to provide opportunity to run KA9Q NOS with simple modems based on TCM3105 or AM7910 chip. I tested the driver only with NOS - no other software was available at the time. Thus I can not guarantee full compatibility with "FTP packet driver" definition.

The driver status can be examined with PKSTAT.COM and the driver can be terminated (removed from memory) by TERMIN.COM. These two utilities are part of packet drivers package from Clarkson and they should work with any "FTP packet driver".

AX25 driver heavily relies on quick interrupt response.  Thus the application software should avoid disabling CPU interrupts for long periods. Or say it in another words: enable interrupts whenever possible.

AX25.COM has several start-time options. To see them start it with "-?". If you start the driver without any command line switch it will use default values: COM1, 1200 baud, etc.

These are the options - default values are in [ ]:

-b<bit rate> [1200] - decimal number

defines the bit rate. Rates below 300 are not accepted because of PC's timer specific features.  Maximum usable rate is probably 2400 but it really depends on your CPU... The upper limit for this parameter is 14400.  "Valid" rates are only numbers which can be expressed as 14400/n.  If you give an "invalid" value it will be adjusted to the closest "valid" one.

-i<software interrupt> [60] - hexadecimal number Tells the driver which software interrupt to use for control functions. Usually you would use numbers from 60..63 here because these are intended for user applications.

-B<COM port I/O base> [3f8] - hexadecimal number

I/O address of RS232 port: 3f8 for COM1, 2f8 for COM2 -I<COM port IRQ> [4] - hexadecimal number   Interrupt request line of RS232 port: 4 for COM1, 3 for COM2 Only the range 2..7 is supported so far. I'm not sure the irq 2 would work on a PC/AT.  After IRQ number you may add letter "p"   (like: -I4p).  This will make the selected IRQ priority highest in the system.  This may help XT users to get the driver   working - the future will verify this.  Normally IRQ0 (system   timer) has highest priority.

-s<slot time> [120] - decimal number 16..255

Slot time for p-persistance scheme. The value is in data bits.For 1200 baud, slot time 120 bits means 100 ms.

-p<persistance> [64] - decimal number 0..255

p-persistance - the higher the value the higher the probability of activating PTT at a time slot.  Your station becomes more "agressive" on the air with increasing persistance and decreasing slot time.

-h<tx header length> [240] - decimal number 8..65535

Number of extra bits the transmitter sends before actuall data is transmitted. This is same as TxDelay on most TNCs.  240 bits means 200 ms at 1200 bps.

You should always make this number as small as possible for best bandwidth use. 240 tells you that many "useless" bits (30 bytes) are being transmitted before real packet data goes out.

Unlike most TNCs this driver sends a square wave not a series of HDLC flags in front of a packet. This was easier to program in software and it makes DPLL lock faster at receiving end.  Thus less header bits should be needed...

-t<tx tail length> [24] - decimal number 8..65535

Number of extra bits the transmitter sends after the actual data.  24 bits makes 20 ms tx tail.

-c<carrier sense mode> [t] - one of the following letters: f,c,t,d mode meaning

f= full duplex - transmit whenever data is pending. This option disables carrier sensing.

c= sense modem DCD line to find out whether channel is busy or  not.  Your modem has to supply this signal. Note that TCM3105 does not do it very well... it sets DCD to high on any noise on audio input. Am7910 is said to deliver much more reliable CD signal.

t= sense data transition - if incoming data signal moves the driver assumes that the channel is busy - BAYCOM 1.5 uses the same (or very similar) way with "CARRIER 1" switch.  Use this mode if you intend to use squelch in your radio.

d= deliver "channel busy" status by analysing incoming data. BAYCOM's v1.5 "CARRIER 0" does effectively similar thing although it uses different algorithm. The driver examines the incoming signals "regularity" - if data transitions comes at regular intervals the channel is assumed busy.  With this mode    you may run your radio with squelch open all the time.  How this option works may depend on modem type. Some modems have still very regular digital output signal even with white noise applied to the analog input.

-T<DCD threshold> [50] decimal number 0..100

Threshold for software DCD (acts only when -cd option is selected) The higher the number the "clearer" the signal must be to trigger "channel busy" status. You should lower this number if your station tends to transmit over others.

-S enable sound effects - when a valid frame is received you will hear a short (1/18 sek) beep from PC's speaker.

Please note that argument line is case sensitive and so are hexadecimal values... "2F8" will not do - you must specify "2f8".

Frequently asked questions with answers.

Q: How to start quickly ?

A: 1. Start the ax25.com - most important parameters are COM base and irq. an example for COM1: ax25 -B3f8 -I4 Now the driver takes control over the RS232 port and stays resident waiting for NOS orders...

2. Start KA9Q NOS (you need the one supporting packet drivers) and then type in: ax25 mycall <your callsign> attach packet 0x60 ax25 5 512 trace ax25 111 you should see packets being received in trace window in most NOS versions you press F9 to get there.

3. Try to connect to another station by typing: connect ax25 <callsign> or split ax25 <callsign> (split window session in JNOS)

This setup in far from complete - it's just to see whether the driver cooperates with your hardware and it only allows you to work native AX.25 Refer to NOS manuals/docs/guides for setting up a complete TCP/IP station.

Q: I use -cd option (software DCD) and my station transmits over others - it looks like DCD status is always "free channel".

A: Try to lower DCD threshold using -T option. Default value= 50.

Q: Are there any programs the driver dislikes ?

A: Yes, SP9AUV discovered a small and nice program saying "Good Morning" (in polish) disabled the driver completely on his PC. The reason is a mistery as the program is not even resident.If the driver does not work, try to start the DOS from a diskette with simplest possible config.sys and autoexec.bat and give the driver another try.

Q: How to change ax25 driver parameters ?

A: If you realized that you have started ax25.com with not the parameters you liked use termin.com to terminate the driver (e.g. termin 0x60) and start ax25.com again with another option set.  At the moment there is no way to change the driver's parameters after the driver is loaded.

Q: Can the ax25 driver be loaded into high RAM ?

A: Yes, the driver can be loaded into UMB to save base memory. However on my 386 when I tried to get UMB using EMM386.EXE the driver performance become worse even when loaded into low RAM. After trying UMBDR521 I got more UMB space (!?) and the driver could run both in low and high RAM without any side effects.

Q: Can the driver work on an XT ? or better: Why the driver does not work on my XT ?!

A: I can't see any reason why not (appart from CPU speed) but haven't got a single report about the driver working on an XT.  In contrary I'm getting lot of claims that "BAYCOM works but the ax25 driver does not". I don't understand why...

Suggested solution are:

1. Try to specify "p" option after -I - this may help.

2. Take KA9Q NOS version from UCSD.EDU - it locks interrupts for shorter amount of time when executing timer interrupt.

Q: How to check whether my PC is fast enough ?

A: Do simple loopback by connecting DTR (data out) to CTS (data in).  Then start the driver at 300 bps and -cf option (full duplex).  Start the NOS, attach the packet driver (see "How to start quickly") and let NOS transmit few packets (for example by enabling beacon every 10 sek). Every packet sent should reapear as received in trace window.  If so try higher speeds, if not either your PC is too slow or something else is wrong.

Q: How big packets can ax25 driver handle ?

A: The receiver can handle 2048 byte frames. This include address, control and data field but not CRC.The transmitter buffer can hold up to about 4KB. That is the amount of data which can be transmitted per each PTT push.

Q: How to modify ax25.com buffer's sizes ?

A: You may want to enlarge ax25.com buffers or make them smaller to save memory. Go into assembler source ax25.asm which should be included in this package - there is a comment about buffers around line 30.

Q: How to compile ax25.asm to get ax25.com ?

A: You type in two DOS commands: tasm ax25 then tlink /t ax25

Q: How to connect my new modem to my RS232 port ?

A: To cooperate with ax25 driver the modem should meet some minimal requirements. Basic roules are:

1. The modem must provide decoded received data on CTS pin.

2. The driver outputs data to be transmitted on DTR pin. Modem should modulate and pass this signal to the radio.

3. Pin RTS controls the PTT of the radio. When it becomes positive the radio (together with the modem) should go into transmition mode.

4. Optionally the modem may provide carrier detect signal on DCD pin.

5. Don't forget about GND line which is the reference for all above signals.

BAYCOM and many other modems build around TCM3105 or AM7910/11 for use with BAYCOM or TFPCX software conform to this scheme.  If you are going to build your own modem you may encounter the problem of level conversion. RS232 ports of a PC use +/- 12 V while most modem chips need TTl levels (0..+5V). The most elegant solution is to use MAX232 or similar converter.  A simpler (but not that elegant) way of doing conversion is to use CMOS inverters (I use 4049 with my TCM3105). Inverters are supplied from 0 and +5 Volts -same levels as modem TTL part. Note that there are no negative supplies anywhere.

The first scheme does work because CMOS inputs accept voltages outside power supply range thanks to input protective diodes. The resistor limits the current flowing through these diodes. You must not avoid it !

The second scheme does work as well because the threshold between logical states in PC's RS232 is a bit above 0 Volts.  BAYCOM team recomends HC or HCT series circuits - they must have a reason for it but I don't know what it is...

Please note that transmitted and received data polarity is not important in packet radio. Only transitions matters. BUT polarity of DCD signal DOES matter: positive means "channel is busy - another station transmiting" while negative means "channel is free - now we are allowed to transmit".

The last scheme shows PPT circuit with simple watchdog timer. It limits the transmition time and is recomended for unattended stations to protect against software failures.

It is a good practice to ground unused RS232 inputs (RI, DSR, DCD). Due to capacitive coupling to neighbour pins there may apear short spikes on them - these will trigger extra interrupts thus loading the CPU.

Warning:I've seen BAYCOM modem schematics with a transistor driving CTS line. This solutions looks good because the transistor's collector is supplied with negative voltage delivered from TxD. But in practice I had nasty problems with such modems when a longer (1 meter) cable was being used... the receiver would not decode at all or it would receive only short frames.

The transistor drives CTS well but only when in saturation state. In open state CTS becomes very sensitive to any pickup - and the main source of pickup is TxD line - there is a square +/- 12V continues wave.  If your cable is long and there is no shield between the wires this wave gets into CTS and makes decoding the true modem signal hard... The problem goes away with inverter drivers or short or shielded cable between modem and PC.

To find out if TxD really comes into CTS run BAYCOM, connect the modem (but no radio) and switch to CARRIER 1 mode (not needed for versions prior to 1.5). If you see RECV instead of QRV you got the problem...	Have fun and please send success/failure notes and comments / hints / complaints to:

email:  jalocha@chopin.ifj.edu.pl

or      jalocha@vxcern.cern.ch

or      jalocha@dxcern.cern.ch

packet: SP9VRC@SP9ZDN.POL.EU (may not work from everywhere)

my home address:   Pawel Jalocha

Rynek Kleparski 14/7

PL-31150 KRAKOW (Poland)

Pawel, SR9VRC

History file

===============

First versions on 25-27 of April 1992: first approach to transmitter code caused inaccurate data transition timing. After fixing that bug another one got in: after transmiting 7KB of data the driver refuses to accept packets.

29 April 1992: More-or-less stable version. Variables aligned to word boudary.

4 May 1992:Defaults for tx head,tail and slot time are like most TNCs

6 May 1992: Tx head, tail, slot time are printed out in bits and ms units.

August 1992: An attempt to cure problems occuring on COM ports handling IIR register not as I wished they did.  By the way minor code review.

November 1992: There was a bug (just by mistyping) in version from August 1992 - when COM interrupt found UART in do-not-need-service state it would not issue proper EOI. This could be responsible for hangups not present in version of May.

-S option (sound effects) added. Every valid frame received makes a 1/18 sek beep.

"p" sub-option after -I added (makes IRQ priority higher)

High-lighting added to some messages. Uses ANSI codes so may cause a bit of disorder when ANSI.SYS is not loaded.

January 1993: Use byte I/O instead of word I/O when writing baud rate into 8250 UART.

-T option - threshold for software DCD (option -cd)

-cd option improved to be insensitive for asymetry in modem's output (bad RXB for TCM3105).

Low pass filter DPLL for clock recovery replaced with random walk DPLL.  I don't really know which is better... random walk DPLL seems to be a bit better... so I leave it in.