BayCom - Terminal Program  V 1.4  for a PC without a TNC

by  Florian Radlherr, DL8MBT and Johannes Kneip, DG3RBU

also Rudi Dussmann, DK5RQ and Christian Lachner, DL5RL

Translation by Denis Godfrey G0KIU.

The Copyright of this English Version is held by G0KIU on behalf of the BayCom Team and may not be stored in any other form without the written consent of G0KIU or the BAYCOM TEAM.

Manual of the Software June 1991

IMPORTANT: The software and/or the Manual(s) must be distributed in their entirety without any Alteration or Modification. Notification of any distribution in UK to be advised to G0KIU @ GB7BAD      It is not permitted to make or Solicit any Charge over and above the Material Costs for Copying.  You are invited to make a Donation to the BayCom Team to assist with further Developments to this Program and      other Projects.

BayCom - Terminal Program Version 1.4 by DL8MBT & DG3RBU (c)

CONTENTS

PART I: Introduction

1.      Forward

2.      Updates to Version 1.20

3.      Overview of the BayCom-Terminal Program

3.1.    How can I make a Quick Start. ?

PART II: Operation of the Program

1.       Installation of the Personal Parameters

1.1.    The Parameter Compiler PARA.EXE

2.       L2 as a resident Programm

3.       The Terminal Programm SCC

3.1.     The New Help-System

3.2.     General Information, Text and Commands

3.3.     Timer- and "TNC"-Commands

3.4.     Description of the Screen Information

3.5.     Commands to Save and Transmit Files.

3.6.     Log book

3.7.     Other Commands

3.8.     Remote Control Operation

3.9.     Unproto-Transmission

3.10.    Short Overview of the Commands and Keys

PART III: Appendices

1.       Error Correction

2.       Overview of the Hardware for BayCom-Software

3.       BayCom - User and Usage Authorisation

4.       Future Developments

PART I: INTRODUCTION

1. FORWARD:

The overwhelming response that we received resulting from the  introduction of BayCom came to us as a big surprise. Now to some extent we have recovered and after a good half year wish to release a new   Version. In many ways this is an improved version, in which errors that existed in V 1.20 in so far as are known to us have been corrected :-  Digipeating, in addition to, for all in Australia and Canada who informed us that V 1.20 had a pronounced aversion to a Call Sign beginning with a V.

Furthermore, completely new features have been inbuilt (See Chapter 2),which for the large part have been suggested by the users of the program. There is nothing new in this Version which goes against our fundamental principles: as before the program is quickly user friendly on XTs, simple to use, undoubtedly pleasing to the user, but is broadly  not so difficult as most Terminal Programs in connecting to a TNC.

Lengthy discussions have taken place, especially with supporters of  prolific Standard Text - but we ourselves have made a firm stand  against this, except for Connect- Quit- and Info Texts, no further Standard Texts are to be defined and that the Connect and Quittexts to be no longer than one line in length. In our opinion it follows that communication on Packet Radio itself should not be reduced to primarily, the exchange of standard froth or Weihnachtstannenbaeumen (Christmas Trees.) for a Greeting. As a smallsweetener it is possible to define for particular stations, longerspecial CTEXTS. (Also a  generated ANSWER-possibility as in Digcom).

A small new introduction, especially for learners, is the comprehensive Help File which can be displayed from within the program. It contains not only important points from the Program area, but displays one of  the most complete Packet Radio Themes Help Sections that are able to be to be assisted. That the Program in other lands has found many friends gives furthermore, the possibility to allow the output of messages in  English from the Program.

For the experienced User, a short list is given of the new additions to the Program, this is followed by the preparations required for starting the Program. Then all the Commands are individually described. The Authors especially thank DL5RL and DK5RQ without whose active help BayCom probably would never have been published, they have given us the time consuming and irksome assistance in the distribution of the Program and Modems, and also with all credit - the corrections. Thanks also to DF1RW for the procurement of parts and all of the OM,  who in many ways have helped, also to you, dear User, who with your  Donations to the BayCom-Fund that has helped us up to the present time to finance 6 BayCom NODES so that a number of different Links have been improved.  Also we are pleased about the many times in the last six months that constructive improvement suggestions arrived here and in the major part are to be found introduced in this Version.

We are annoyed by a number of individuals both at home and abroad who by the sale of our Hard- and Software saw the possibility of filling their own purses. What chances for progress in this Operating Mode, if these people turned their copying energy to more constructive objectives. !

It is reassuring to us however, that these portrayed occurrences excepted, and set against them, are the pleasant contacts with many BayCom-Friends and the many commendations received with the origination of the new Version. With Greetings from the Bavarian Countryside.   DL8MBT, Flori (@DB0AAB), DG3RBU, Johannes  (@DB0RGB)

2. NEW UPDATES TO VERSION 1.20

For the experienced User there follows here, only the Keywords with which the updated Program is concerned. The page number is also given where precise Information of the respective information is able be    Found.

- Operation with AM7911-Modem is now possible with 300Bd, HBAUD

- System Messages are switchable German/English

- Log Book on Diskette or Hard Disk, choice of simultaneous Logging with the Connect or at the end of  the Program; selected with the Command LOG and EXITLOG

- Establishment of a Disconnect-Timer (Timeout) for an inactive connection

- Switchable between Insert and Overwrite Mode :INSMODE

- Restoration or Clearance of the DOS-Screen now possible

- Remote Control Operation within limits, of defined Commands

- Display of 50 and 60 Lines with a VGA Card possible

- Bell and line "knacks", also L2-Flash switchable (Now fully office usable !!)

- Personalised Connect Text available  (:ANSWER-Function)

- A one line definable Quit Text.

- Connect-Call in the lower Status line updated by "further connection"

- Various smaller alterations, e.g. Reduction of the Scroll back line count in the TX window, therefore more scroll back lines available in the RX window

- Issue of the Time at Connect/Disconnect

- Issue of the time before each Packet in the Monitor Window

- Establishment of a Command-Order (: in the next line following the issue of a Command; automatically set or not set)

To replace Version 1.20 with Version 1.40, all Program Parts of the old version must be replaced by the new Files (also SCC.INI, PARA.EXE etc). Compared with Version 1.20 General Operation, nothing is changed.

3. OVERVIEW OF THE BayCom-Terminal

GENERAL ATTRIBUTES:

- Partitioned in a "TNC-Part" and a Terminal-Part. The TNC-Part runs resident and unseen concurrent with other Programs under DOS in the background. The Station is then available for Connection. The      incoming information is obtained by the User on the Screen, only after the start of the Terminal Program.

- Screen Editing in the Terminal: Each output of the Program is able, by movement of the Cursor to the required part and followed by a <CR> to be re-transmitted.

- (3) part Screen: TX, RX and Monitor. The Cursor is capable of being positioned in all three windows, the Windows are able to be altered considerably in size, the Text scrolled backwards and forwards.

- Previous pages of all Screen Windows capable of being Scrolled.

- Usual Multi-connect-Availability including File Transfer.

- Support for EGA and now also VGA Monitor Modes.

- Saving and restoration of the Screen Contents after an exit from the Program.

The BayCom-Terminal consists of Multi part Programs, which will be explained in the following sections.

L2.EXE. This is the central Program Part of the BayCom-System already mentioned above. This Program is of great importance: It runs resident in the RAM. That means that after the Start of the Program (L2) the customary DOS-Prompt is present, a small Flashing Rectangle is to be seen in the top right corner of the Screen as an indication. The Computer with a Modem, and TX/RX connected behaves as a free standing     TNC: It is able to be connected to, receive input information, but the user can only see this after the Terminal-Part has been started (SCC.EXE). When L2 has been started and the DOS-Prompt displayed, the Computer is available for other work - e.g. Word Processing,  Programming etc. by selection, with a reduction of RAM space: L2 occupies approximately 90kB of the available RAM, i.e. 640kB is no longer available but 550kB (From that another 100kB for the operating system is also unavailable.) This can cause Problems when RAM greedy Programs are run concurrently with L2.

SCC.EXE. This is the User Interface. After the start, a 3 Part Screen appears, the Operator is then able to perform a complete Packet Radio operation. From Digicom the Program has inherited a Screen Editor.       For the Commands see the descriptions later in the Manual.

OFF.COM. This short Program serves as a "RAM Charwoman". It removes L2 from the RAM. The Computer is then fully available for other Tasks. Exception- if other RAM resident Programs (e.g. Norton       Commander) have been started after the start of L2, see detailed Info under OFF in Help.

SCC.INI. This is a normal ASCII-File which contains all of the User Default Parameters.  It is able to be Edited with a normal Text Editor. In this File the appropriate Calls and Parameters are placed.

PARA.EXE. The Parameters in SCC.INI must be converted from ASCII into a Machine Readable Format. This conversion is performed by PARA.EXE and is called automatically by the re-start of L2 after every alteration of SCC.INI. However it is able to be started manually. The Program gives an Error Report in the case of an Error in SCC.INI.

SCC.PAR.  This is the File created by PARA.EXE from SCC.INI and cannot be manually altered. It is created after the first call of  PARA.EXE.

TERMHELP.SCC.  The Help-File, containing the Text, which is called after the issuing of :HELP and displayed on the screen.

SCC.VID.  BayCom is capable of saving the Screen Contents at the end of the Program, and at the next Start, re-loading the stored Contents to the Screen. The Screen Contents will be stored in the File      SCC.VID. The initial creation of this File follows the first Start of BayCom.

SCC.PWD. Password-File  for the  :J-Function (must be user created, see :J-Function). IT IS ESSENTIAL TO CREATE THIS FILE EVEN IF NOT USED.

SCC.LOG. A Logbook-File will be activated by LOG-Command (see P. 44) from the Program.

BAYCOM.BAT.  With the calling of BAYCOM, L2 will be started and subsequently the Terminal (SCC.EXE). At the end of the Packet Operation L2 will be automatically removed (OFF.COM), thought to be important for the User, who will have no further use for a resident L2.

INSTALLC.BAT. This is a small Batch Program that will create a Directory named BAYCOM on the Hard Disk (C:) and then copy all the files from the Diskette to that Directory.

3.1 HOW QUICKLY CAN ONE BEOME QRV ?

For all those who only require a few short instruction and afterwards prefer to set about testing for themselves, the following lines are offered. BayCom is usable from either a Diskette or the Hard Disk, in the latter case all files should be copied to a chosen Directory. (Or you can use the Batch File INSTALLC, which will create a Directory BAYCOM on C: and then copy all the Files into it. Information has been distributed on Mailboxes recently that BayCom will only run out of a particular Directory.  This is not correct.  If you are updating from Version 1.20 the entire set of files must be cleared and the Files of Version 1.40 copied into the Directory. It is only necessary to change a few Parameters. With a normal ASCII Text Editor the following Parameters should be installed in the Data of SCC.INI. (That is a normal Text Data File.)

- The Port COM1 or COM2.  This is defined in SCC.INI around Line 30 and is pre-installed as Standard on COM1.

- The Call Signs. The Parameter MY (MyCall) is to be found around Line 70. The individual Call Signs must be installed, also the Digipeater  (DCALL).

All the other Parameters are set at tested Values but can be changed to suit your taste, after experience has been gained with usage.  Next L2 must be started, followed by SCC (Or better still altogether through the use of Batch File BAYCOM) .....and already all should be running !  Hope you remembered to connect the BayCom Modem and rig. !!     To end the Program ALT-X.

DO NOT FORGET: As with Digicom in front of each Command a Colon is required. This can be obtained by ESC, TAB or DEL. When all else fails and nothing works then find a quiet place and read the following instructions.

PART II: OPERATION OF THE PROGRAM.

Fundamentally the Basic Operation of the Program has not in itself altered. Friends of 1.20 or of Digicom will become quickly accustomed with its operation. For all of those, the new update procedure is yet once more described.

1. Install the Personal Parameters in the Text File SCC.INI (only important for first start or large Parameter alterations.) Thereby setting the Call Signs of the Station, Screen and TX Parameters.

2. Start the resident L2, this Program allows the sending and receiving of Data. (If SCC.INI has been altered then automatically the updating of the Machine Readable Parameter file (SCC.PAR) will take place.)

3. Start the Terminal SCC (This is the User Interface enabling the user to Read the incoming Data and Enter Data for Transmission. L2 having been already started may have information to display once SCC is running.

4. Ending of the Terminal Programs (ALT-X)

5. Ending of L2 (OFF)

The items 2, 3, and 5 can be achieved by the use of Batch Program  BAYCOM and then these items will be run automatically.

Following:- individual items will be explained. Before using the Program a copy of the Diskette should be made, or copied to a Directory on the Hard Disk of the Computer. The Batch-Program INSTALLC will do     this automatically, creating a Directory BAYCOM on the C: Hard Disk and then copying all the files into that Directory. It is also required that the Program runs on a 100% Compatible IBM-PC.  (Compatibility is especially important for the Serial Port.) with an Operating System > DOS 3.0 (With DR-DOS problems occur with the RAM management when using the :OSHELL-Command).

1.  INSTALLATION OF THE PERSONAL PARAMETERS

Before the first Start of the Program the important Parameters must be installed in a completely normal Text File. (Named SCC.INI) This Text File can be edited with any favourite Editor. (e.g. NE, Turbo Editor     etc.)  However Caution is required with the Text Processing of the File :- Formatting - Values or Text in the Correct place after a Command - Character count in the text Entries etc. In the following, the individual lines of this Initialisation File will be precisely described by the means of a printout of the File SCC.INI with   explanations annotated.  Everything following a ; will be ignored by  the Parameter Compiler and will be treated as a Comment. It is strongly recommended that before beginning any alteration of the File that a     copy is made of the Master File; so that in the event of serious problems at a later stage, a  Valid File is still available.  WARNING: The altered Data for the Terminal will only be passed to the Machine Code Program when L2 is re-started. Consequently it is not useful to alter the File whilst L2 is running and afterwards start SCC,   first L2 must be removed by running OFF and then restarted. Here in the introduction to the explanation of SCC.INI, the text that stands between [  ] is not included in the actual File but only used for     tutorial purposes. It is also permissable that all other Commands, which are described in this tutorial, to be utilised in SCC.INI.  In the event that the user prefers other values to the Standard Values given here, these parameters are written in an optional place in SCC.INI.

1.1.  THE PARAMETER COMPILER PARA.EXE

In previous paragraphs it has been described how in the initialisation of the Program it is able to employ the use of the Parameters installed in a Text file. When the alteration of the Parameters is completed, in order to reduce the RAM requirement these Parameters must be converted into a Machine Readable Format. (e.g. the Computer requires no Commentary from the SCC.INI). This Compilation happens completelyautomatically after each alteration of SCC.INI (will be known from the Program Date and Time) at the Start of L2. However it is possible, to decide if the SCC.INI has been correctly altered. The "Parameter Compiler" is able to be manually started by PARA <Return>. On the Screen it will be indicated how the Program processed the lines and eventually to give an error Report. (if PARA has been manually started, then L2 will not call the Compiler at the next Start as the correct Machine File is already available.) The Compiler produces the now Machine Readable File SCC.PAR, from which L2 and SCC can obtain their information.    In the case that an ERROR is issued, one should prove the following:

- Does all Commentary stand behind a ; or has a semicolon been erased ?

- Has a typing error or an invalid value been given?  If Errors are reported, the Line that is in error will however be set independently with a suggested Value, a Program Crash is therefore not to be expected.

WARNING: In addition, practically all of the Parameters are able to be altered from the Terminal (except the basic initialisation, e.g. Video-Card etc.). These Terminal alterations will be saved in the Machine     Readable Program SCC.PAR. This means that they will be used repeatedly unless the Text File SCC.INI is altered when a fresh compilation will take place. Unless the previously mentioned Terminal alterations are     made in SCC.INI they will be lost. So it is important that major Parameter changes are made in SCC.INI.     Most Commands can be altered from SCC (Terminal Prog.) but will only become effective after a re-start (e.g. VADAPTER).  A further new Function of the Parameter Compiler is the Indexing of the File TERMHELP.SCC, which is in the complete Help text for the comprehensive Help Menu of the Program. The call for this is PARA -H,  further Information is contained in Chapter 3.1.

2.  L2 AS A RESIDENT PROGRAM

L2, the Central Part (L2 Stands for Layer 2) of the BayCom-System appears in an otherwise earlier neglected but often employed Program Form for PCs, which Packet Radio Amateurs know as a RAM resident Program. It should be understood that once the Program has been startedit runs in the background and a Normal DOS Prompt is then shown. The Computer is now available for other Operations. L2 is started by the Command L2 <Return>; a short switch on message should then appear followed by a flashing rectangle in the top right hand corner of the Screen, followed by the normal DOS-Prompt re-appearing. Internally the following occurs: The Program reserves for itself a part of the Main RAM of the Computer. This area is not then available for subsequently started Programs. In this RAM section L2 makes itself very comfortable and works very industriously, it decodes Packets and in the meantime stores detailed Information. In order to do this it naturally requires CPU time ... to achieve this it makes an Interrupt every milli second

- The Processor interruption therefore causes any other Program that is running to be slowed and in some cases they can be incompatible.

What does this mean in practice ? In the first place the User does not observe the continual Interruptions from L2 once it has been started. This is because these interrupts are furiously quick in operation. In     principle other Programs are able to be started, e.g. a Morse Program, Word Processing Program or similar. To observe is, that the Computer now has less free RAM than it had before the Start of L2. L2 grabs    around 90kB of the 640kB of RAM. This means that RAM intensive Programs are no longer able to be run. In order to see how much RAM the System now has available, the DOS-Program CHDSK (or similar.) should run after  L2 has been started. The running Data and the free RAM information is then obtained. This is not the complete difference between 640kB and L2.. in other areas of the RAM there are the Keyboard Driver and the System Programs - CHKDSK without L2 started will yield this value.  A second impediment to yield eventual Problems with other Programs is naturally, that not only RAM, but also Processor Time with L2 must be considered. This can mean a small loss of Program running speed, especially when a lot of Data is being received and L2 has a large quantity of work to process. Most Programs will not tolerate this and therefore Crash, sometimes a Program creates a load of old rubbish in  L2 RAM reserved area .. and it then insidiously clears out L2. In the second case it means that simple tests should be made, as to whether L2 and the additional Program canlive together or not. Whilst L2 is active there is a small flashing rectangle, this serves simply as an indication that the Program is active. So long as this rectangle flashes, L2 is not corrupted (Otherwise the flashing will stop; in a Program that switches into special Graphic Modes, the flashing should be visible again as soon as a return to DOS is made). The flashing however is also able to be switched off, in the case where it disturbs other Programs: Command :BLINK ON/OFF in the SCC.INI.  This Flashing is able to be used as a Function Monitor: If it shows,  all is in order, BayCom should then run correctly in the Computer. If  it does not, see the Chapter Error correction in the Appendices.  If L2 is running, the Computer with a Connected Modem and Radio is  absolutely similar to a Computer with Radio and Free Standing TNC. That  is, one can connect from an external Station and leave information. The Host Station User at the Keyboard is not aware of this happening so long as the Terminal SCC has not been started, when SCC has been started the information that was accepted from the time that L2 was     started becomes available.

A resident Program naturally is not able to be ended simply and removed by the press of a Key, it should be made possible for the Computer without having to be switched off and then on again, to be instructed to remove L2. We offer therefore a small Program as a"Speicherputzfrau"   (Storage Charwoman.!!), namely OFF.COM, to be calledby OFF. This clears L2 out of it's reserved RAM area and the Computer returns to normal.

3. THE TERMINAL PROGRAM SCC

As in the foregoing description, L2 serves as the Receiver and Transmitter of Packets, through the accomplishment of Connect and Disconnect. Consequently to the complete development of the underlying    levels of the AX25-Protocol, it now requires the USER-INTERFACE, which will display the received and transmitted data. The SCC.EXE serves as this vehicle. This User-Interface as in Digicom is again full Screen orientated, with which the acquaintance of the Program was very simple.SCC.EXE is started by issuing SCC from DOS (L2 must have already been started). Then the required three part Screen becomes visible. The (3) Windows are separated by two Status Lines which in the  following will be explained.

Upper Status Line:    QRV  DL5RL> Disconnected mem=038 ln=203 n2=10 o=0 fr=20 k=0 16:34 2 I U     1     2         3         4       5      6    7   8     9    A   B C D

The individual points:

1 Here will be displayed, the Mode in which the System finds itself.

QRV:  Receiving, but no signal (Closed Squelch)

RECV: Receiving incoming Info. (Squelch Open)

SEND: Data is able to be sent immediately.

If the ability to transmit is prevented by the pressing of SHIFT-TAB or + in the Keypad area, or if the Cursor is located in one of the lower Windows, the blocking of the  transmission will be signalled through     the FLASHING of the QRV/SEND/RECV.

2 The individual Call Signs (with SSID) will be displayed here

3 Immediate state of the Port, the following possibilities exist:

| Disconnected |  |
| --- | --- |
| Link setup | Connect set up in progress. |
| Disc request | Connection will be broken |
| Info Transfer | Information transmission |
| Frame Reject | Serious Protocol-Error occurred |
| WAIT ACK | Poll-Packet transmitted |
| Reject sent | Info. with unmatched Number received |

mem gives the information about the Quantity of immediately free Buffer of the System. This can set by choice in SCC.INI.

ln gives the quantity of Lines which are able to be Scrolled. The size that ln is, depends upon the size of the RAM working space of  the Computer and the number of active Ports.

n2 is the actual state of the Retry-repeatable count. If this goes to 0 then the Connection is aborted.

o is the Number of Packets which have been sent in this Connection, but not yet Acknowledged.

fr is the Value of the immediate Frack-Time on this Link. (By Frack it should be understood that it is the time which has to be waited before an unacknowledged Packet is sent once again. Frack adjusts itself according to the Band conditions.

k gives the Channel Number on which the Port sends it's Data. With the RS232-Version it is always 0, This is relevant only to the Multi-Channel System with the Expansion Card.

The actual Time as taken from DOS

The immediate Port Number is given here

I  stands for Insert-Mode, R for Overwrite Mode for Text Correction on the Screen. Able to be toggled by means of the INS key.

U This indicates the actual Status of the Unlaut Display. U indicates that the IBM-Umlaut will be displayed.  This is not significant for Amateurs who are not using a German Keyboard Driver or not typing German Language.  Able to be switched by ALT-U

The Lower Status Line contains the listing of all the Ports and the immediately connected Stations or those that are in the process of  being connected.  If one is Connected to a NODE/DIGI and then by way of  a further Connect Output to set up further Connection, the Port  Indicator will be updated to this Call Sign. Similarly it is effective for eventual Reconnect-Reporting.

The number of Ports is able to be set in SCC.INI (Max.8). At the end of this line a File Transmission or Reception is indicated together with the actual position in the File. (Received or Transmitted Byte Count.)  If new Information arrives for a different Port to the one selected, then the Indicator for that Port in the lower Status Line will Flash until such time that the Port is accessed by use of the appropriate Function Key. F1-F8.   If the Status Line does not show the previously explained Information check the Colour Combination (SATTRIB in the SCC.IN File.) white letters on a white background are not a good contrast. !!!

The Program responses are able to be switched to German or English,  this is achieved by the switching of  :EN ON or OFF (EIN or AUS) The Programs OFF, L2 and PARA give their responses  always in German.  It follows that through the Calling of SCC by the issue of SCC -C the Screen will be completely cleared for the start of the PR  op.

SPECIAL KEY FUNCTIONS

The individual Ports are able to be changed by the Function Keys. F1 selects the first Port, F2 selects the second Port, F3 selects the third Port and so forth.  F10 selects the Monitor Port, this gives a larger Monitor-Screen.  F9 moves the Cursor from the TX window into the immediately underlying  Screen or to the TX window from the Monitor Window:- on the Normal Ports one press into the RX Screen, two presses into the Monitor Screen. Then after a further press, back into the TX Screen. From any of the Screens a Press of one of the Port Function Keys will return the Cursor to the TX Screen of that Port.

The Position of the Separating Status Lines is able to be moved as you please, and by means of the simultaneous pressing of CTRL and HOME (7 on the Numeric Pad) the Upper Status Line will be moved up, and with CTRL and END (1 on the Num.Pad.) moved down. The Lower Status Line will be moved upward with CTRL and PgUp (9 on the Num Pad.) and moved down with CTRL and PgDn (3 on the Num. Pad)

A similar Function to F9 is also available as follows :   (1, 2, 3. are the keys in the Num. Pad)

ALT and 1 moves the Cursor to TX Window

ALT and 2 moves the Cursor to TX Window

ALT and 3 moves the Cursor to the Monitor Window

One should be aware that the TX Window at the top of the Screen must  have a minimum of 2 Lines, or eventually Program Reports will not be received. !  F10 moves the Cursor into the Monitor window, which is enlarged by removing the RX Window, leaving just the two Windows (TX and MON.).  All Information, which by Scroll back is displayed on the Screen, is again able to be made visible, in the individual Window where the Cursor is placed. (Function Keys!) Then with PgUp or PgDn move backwards or forwards. It is able, dependent upon the Number of Ports and available RAM to hold (buffered) approximately 200 Lines per Port. WARNING: So long as the Cursor is in the RX Window, the display of incoming text is halted. It will appear when the Cursor has again left the RX Window. (Distinguishing Sign: Flashing of the RX Indicator!).  The Output is able to be halted by choice, namely through pressing  SHIFT-TAB Keys or by pressing the + on the Number Pad. Once again the RX Indicator will Flash in the Upper Status Line.  The Program is able to be ended with ALT-X.

CTRL-Characters (e.g. CTRL-Z) are able to be displayed only on the Screen and not Transmitted. As normal, you are able with the Return-Key to Transmit.

INS-Key Switches between Insert Mode and Overwrite Mode for Text Input,  the state is shown in the Upper Status Line.

ALT-U will switch the Umlaut state from DIN-NORM or IBM-NORM

CTRL-F1 combination will obtain a Help Text (if available) at the position where the Cursor is placed.

ALT-F1 gives the previously displayed Help Text once again.

Here once again is a Short Collected Version of all Key Operations:

| F1 - F8 | Switch to the Corresponding Port. |
| --- | --- |
| F9 or - in the Number Pad | Cursor to the next lower Window. |
| F10 | Monitor and Unproto-Window |
| ESC or TAB | Switch between Commands and Data, sets or clears a : at the beginning of the Line. |
| SHIFT - TAB | Stops the output on the selected Port. or + in the Number Pad |
| CTRL - RETURN | Cursor moves to the next line, without the line being exploited (as SHIFT -RETURN in Digicom) |
| PgDn | Text scrolled one Page downward (Forwards). |
| PgUp | Text scrolled one Page upwards (Backwards). |
| CTRL-PgUp | Monitor Window enlarged, TX Window Reduced. |
| CTRL-PgDn | Monitor Window reduced, TX Window Enlarged. |
| CTRL-Home | TX Window Reduced, RX Window Enlarged. |
| CTRL-END | TX Window enlarged, RX Window Reduced. |
| CTRL-F1 | Help Text according to the Word under the Cursor. |
| ALT - X | Program SCC.EXE ended |
| ALT - 1 | Cursor placed in the TX Window. |
| ALT - 2 | Cursor placed in the RX Window. |
| ALT - 3 | Cursor placed in the Monitor Window. |
| ALT - U | Umlaut toggle |
| ALT - F1 | Re-Display of the previous Help Text. |
| INS | Switch between Insert and Overwrite Modes |

3.1. THE NEW HELP-SYSTEM.

One of the most important additions to this BayCom-Version is the New Help-System, especially for the inexperienced Packet-Radio User. The idea of this innovation is not only to be able to obtain information quickly about Problems with the Program thus avoiding long leafing through the Instructions, but also to give Help in situations; e.g. in the use of Digipeaters/NODES. The Help Text contains not only BayCom specific Information, but also a complete range of explanations of all Packet Themes, such as Timer setting, Mailbox advice and others. Finally the Help Text is also able to be enlarged and completed by every user. How that functions, will be explained at the end of this Section, but next is an Overview of the  Service given by the Help Text.

HELP This Command (:H) opens a Window on the Screen, in which the New Help Menu will be shown. The Help is arranged in different Sections, e.g. in Screen, Keys, Modem, Terminal-Commands. These items in the Help -Windows are highlighted and are able to be individually selected by use of the Cursor Keys. (Position brought into prominence) A press of the Return key brings further ideas about the chosen subject into view.     Also in this Text once again items are Highlighted, and under these items further information is to hand.  Therefore it is possible to wend and weave through this Help Structure. The "Z" Key returns to the previous Window, with "H" to the First-Help-Menu; and with the "E" key or ESC-Key to exit from the Help-System and return to the part of the Screen from which Help was called.  More pages of Text for an individual Keyword are able to be obtained by the use of PgUp and PgDn Keys.  (As indicated in the lower right hand corner of the window when available).

A further option is with the Command :-

:H <Keyword>

Complete Help for a particular Word or Command is able to be selected.  If a Text is available in the File, then it will be displayed in the Window by this Command. An Index of the available Keywords is also able    to be called, if whilst in the Opening Window of HELP (Which was called by :HELP) the Keyword :INDEX is selected. In the event that one has the notion when in the middle of existing Text to obtain Help for a Word that is in the Window, then the Cursor should be placed on that Word and the combination Ctrl-F1 pressed. If a Text is available it will be displayed in the Window. If the word is not available as a Keyword, a search will be made to see if the word is available as a part of another Keyword. If it is, then the Text belonging to this Keyword will be displayed.

EDITING OF THE HELP-FILE TERMHELP.SCC:

In principle it is handled by the use of an Editor in the same manner as a completely normal Text File. A list will appear at the beginning of the File in which the Keywords and a Number (Place of the Keyword in the File) are presented. The individual Help Texts begin after an appropriate advice. These Texts are able then, to be altered as one wishes with the Editor, provided that certain rules of the game are observed. Thus for example, the Index list at the beginning of the File may not be edited, later it will be automatically produced by the Help Text Compiler PARA.EXE.  In the individual Help Texts the Keywords and Cross references must be identified. Each Keyword may be up to 10 Characters in length (all others will be ignored) and be identified by the preceding  \\ (two backslashes), e.g. \\HELP.

A Cross Reference, also a Keyword, that will be Highlighted later in the Help File, and enabled by selection to make a leap to the next Keyword, will be identified by the following means that which stands between carets (^), e.g. ^SCC-Card^. Naturally when a Cross Reference is to be made, there must also be on hand an Index Word, this fact will also be automatically checked for consistence by the Compiler.

For the correct output to the Screen Window certain Formatting of the Text must be observed and information must always be written in Pages of 18 Lines of 69 Characters. The last Page (also with 1 page the first as well), may nevertheless, have a maximum of 17 lines. (If disregarded an Error Message will be issued: "Unguenstige Seitenlaenge") [Invalid Page Length].      {Translator found that it was only possible to get 66 Characters per Line.}

The Pages should be separated from each other by simple Empty Lines, which for reasons of Legibility are certainly better for an empty line above and below. For a Two Page Text the Lines 1., 18. and 19. should remain free, Empty Lines after the end of the last Line are not important and lead to an Error Message, if it leads to unnecessary re-organisation. If a Line is longer than the standard 69 characters, the rest will discarded.  Contents in the Text may be any Character  ^ and \\ will be exclusively interpreted as Commands and with the exception of the required place, may not be found anywhere else in the Text.

When the Editing is finished, the previously mentioned Index must be brought up to date once again.  This is achieved fully automatically by the now familiar Parameter Compiler PARA.EXE and must be started as  follows:

PARA -H

(H stands for Help File). Organises the Index, except for the output of collected Error Messages (Format transgressions, missing Keywords for Cross Reference and similar.) It will find places within the expanded list of Keywords. Between the Index and the actual Help Text there is arrayed a number of Dummy Lines. (containing *******).   Should this reserve disappear after enthusiastic activity, fresh ones should be inserted once again.  (Empty Lines are not sufficient !!!) otherwise the beginning of the Text will be relentlessly overwritten !!!

AFTER PARA -H <CR> THE NEW HELP FILE IS AVAILABLE FOR THE FIRST TIME. (the normal PARA-Runs after alteration to the SCC.INI but not for this)

We wish much fun with the Editing of the File, but we request though, that MegaBytewise to avoid on grounds of Net Loading the placement of TERMHELP on the PR-Network. Send your Supplements and Alterations not the complete File to G0KIU (Address in the Appendix) who will integrate these into the TERMHELP.SCC as long as there is room on the Program Diskette.

3.2 GENERAL INFORMATION ABOUT TEXT AND COMMANDS.

In usage BayCom will be relatively very quickly understood especially by Digicom-Users. Because it has certain indigenous Standard Commands, the Authors hope that Users changing from other Programs, and without many "Manual Lectures" will become  quickly proficient with its operation.

Fundamentally it follows that:

-Each Command will have a ':' at the beginning of the Line and with a Press of RETURN be activated. All Lines, which are completed with RETURN and with no Colon at the beginning will be treated as Text and  sent to the opposite Station. If no Connection exist, then such Output on the Normal Ports will be swallowed. From the F10 Window and issued without a ':' Text will be sent as Unproto-Text. (For explanation see Corresponding Section).

The Commands, which in the following are given a Valid ON / OFF possibility, are also always fully valid with EIN or AUS.

- Upper and Lower Case Characters are basically not relevant.

- Commands are able to be issued from any part of the Screen, therefore also from the TX- and the Monitor Windows. The only requirement being the ':' at   the beginning of the Line.

- In addition the ':' is able by Pressing the ESC- or the TAB-  Keys to be set at the beginning of the Line. A speciality is therfore, that by a Press of one of these Keys from any Position of the Cursor in the Line:- to be able to set the : at the beginning of that Line.  (The Line contents will be moved one position to the right). The ':' is able then by a renewed Press of either of the Keys to be removed, again the Cursor can be in any position on the Line.

- Commands are able to be shortened, the Combination for each Command is shown IN CAPITALS in the following explanations.

- A Command, which is issued without the respective following Parameter  (i.e. EIN, AUS, ON, OFF or a value), shows the Present Value.

- BayCom similar to Digicom is fully Screen Editor Orientated, i.e.  Commands etc, are able to be issued from each Window, in order to issue the Lines that are already on hand a RETURN must be given.

- It is not necessary to hit the return Key at the end of each line in order to send the Text to the opposite Station. The last word automatically wraps into the next line and the completed line is transmitted.

- Most of the Parameters will be pre-set in SCC.INI. The given Default Values current for the SCC.INI contained on the Distribution Disk are considered proven Values.

- Control Characters of the opposite Station will be displayed on the  Screen.  CTRL-G will sound the Bell of the System.

3.3  TIMER AND TNC COMMANDS.

:BAdress<End C/S><Sender C/S><Digi1><Digi2>... Gives the Target, Sender and Digis of the Beacon. Example: :BAdress DK5RQ DG3RBU-8 DB0RGB sends the Beacon with the Sender's C/S DG3RBU-8 to DK5RQ via the Digi DB0RBG.  A Beacon will now be transmitted if :BEACON is not set to 0.  (See the following)

:BEacon <Value in 1sec-Units.>    (Installable values 0 .. 32767)

Defines the intervals between Beacon Transmissions. Beacons will be transmitted as Unproto. Sender Call Sign and Addressee will be determined by :BAdress, and the Text of the Beacon in :BText  It is advised that in order to keep QRM on the Frequency to a minimum, that only to set it, if it is unquestionably important (As a rule only for Test Purposes) and also then only with the largest possible intervals. :BEacon 3600 will send the Beacon at Hourly Intervals. The Beacon is switched off by the setting :BEacon 0. The Beacon is able to be defined on Multi-Channel Systems for each Channel individually.  Then the Channel Number must be set before the Value. (:BEacon 0 1800)

:Btext

The Text that will be transmitted in the Info Field of the Beacon.

Example :- :Btext Linktest, sri for the QRM

:CArrier <CHANNEL> <Value>

This Command is only Valid for the Utilisation of the 8530 Expansion Cards and will be ignored with the Use of the RS232 Modem. It allows, on each Channel a Hard- or Software DCD Operation.

0 = Soft-DCD (Squelch is able to be left OPEN)

1 = Hardware-DCD (DCD-Control of the 8530 will be addressed and

must be able to be served from the Modem.

2 = Hardclock + soft - DCD

3 = Hardclock + hard - DCD;

4 = fullduplex  Fulllduplex-Operation

:CNot <Call> <Call>...

Sets a list of Call Signs from which no Packets will be accepted. It will not then of course, send a DM, but Packets from and to Stations in the List will be handled, as if they were never received, therefore simply ignored. The Packets however will be shown in the Monitor.

:Connect <Call> via <Digi1> <Digi2>

The Connect-Command builds a connection to the end Call, via the given Digipeaters. It is permissable to include up to 8 Digipeaters. For Direct Connection naturally only the Call Sign of the QSO Partner is   required. (e.g. :C DF1RW). The "via" is able to be substituted by a Space or Comma. If :C or :CONNECT without a Call Sign is issued, the Call will be issued to the immediate previous Call Sign on that Port. So if it is required to re-connect to this station, move the Cursor into an empty space Line and press RETURN. By this method one is safe from unpleasant surprises. If a Connect is sent that was mistyped, then it is not necessary to issue :Disconnect, the Command is able to be overwritten followed by  <RETURN>. The false Connection attempt will the be stopped and a :Connect request made to the Correct Station. When using the BayCom Expansion Card with additional Channels, the Channel is able to be selected after the Target Call Sign. The possible Values are 0..7 (Or equivalent A..H). If the Command is issued from Port 10, a connection will not be attempted but the Unproto-End-Address for UI-Transmissions will be set.

:Disconnect

This Command severs an existing Connection or aborts a Connection attempt. If the Command is issued twice one after the other, a reply will not be awaited from the Connectee (UA or DM) and the Port will be returned to the Disconnected State. This is important, if a Connection either through a failure of the Partner or the next Digi, is broken, it is not necessary to wait for the Retries to run out.

:DCall

This is the Call Sign through which the Station Digipeats Packets. DCall is similar to MYcall but must not be identical.

:DISCtime <Time in 10 Second Units>   (Valid Values 0 .. 600)

This Time gives the so called Timeout, that is the automatic severance of a Connection, by way of the length of Time that no more Information Packets have been conveyed. With :DISCtime 0 the Timeout will be switched off, :DISCtime 120 means severance after an inactive 20 Minutes.

:DWait <Value in 10ms>      (Valid Values 0 .. 250)

Waiting Time for which the Channel must be Free before transmitting is commenced. The actual DWait is a contingency Time between 0 and double the actual installed Value. AN OBSERVATION: In recent times it is the case that in order to obtain an advantage sought by OM over others OM; a  use has been made of a very short DWait. In hoping that co-operation one with another on the same Frequency will occur, we request DWait to be so installed that other Stations have a chance to be able to Transmit. A fair DWait-Value lies between 10 and 30 (100ms - 300ms). It is a fallacy to believe that with a shorter DWait it is able to get through quicker !!. DWait with the operation of Multi Channels must have the Channel Number given    before the Value, with only one Radio Channel it is adequate to give just the value.

:Frack <Value in 100ms-Units>    (Valid Values: 10 .. 200)

Indicates a Start Value for the repeat Output of a Frame (or a Poll), if after the previous Transmission no acknowledgement has been received. The installed value in SCC.INI (or from the Terminal) is only a Start Value, which adapts itself during the course of the Connection by using the throughput answering time of the Partner, it is shown in the Upper Status Line (round-trip-timer). Sensible Start Value 40 .. 60

:HBaud <Channel> <Value>

This is the Baud rate for a Channel. When it is utilised with the RS232 Modems the Values 300 and 1200 are possible (SW/VHF/UHF). All other installed Values are ineffective. With the utilisation of a PC-Expansion Card the Baudrate is able to be installed quasi stepless. (Quantification in straight fractions of  38,400Bd).

!!! WARNING: After the alteration of HBAUD one Packet must always be sent, then the Installation will be operable. For 300Bd modulation on HF the normal TCM3105 -Modem is not able to be utilised, it must be a Modem with the AMD-World-Modem-Chip AM7911 fitted!!!

:IPoll <Frame length>     (Valid Values 0 .. 255 (0 = OFF, advised = 80))

Info-Poll should be understood to be the immediate re-iterative sending of a short Frame, in the case that the opposite station has not been able to make an acknowledgement. Although in accordance with AX.25-   Protocol in such a case of a non-acknowledgement, a Poll (by way of RR) is to be sent, this in itself has in the past been accomplished by the Info-Poll. It is therefore suitable, with short Frames to be able to save a lot of Overhead (therefore to only send once instead of twice for a non received Packet). It reduces the  consequent collisions and allows a flowing and faster running connection. Today Info-Poll is dominating all Software Versions. The Number after :IPOLL indicates at what length of the Info-Field in the Frame, an Info-Poll should occur, whatever goes out , to be handled completely normally for  AX.25. A suitable value is for example, :IPOLL 80, this value is approximately where the statistical barrier lies between total overhead and profitable Data flow.

:Kanal <Value>

(Only for Multi Channel Systems with Expansion Card) Toggles a Port on a special Channel. In the case of a Connection that is live, then this is not valid. The change is only valid for the immediately selected Port, thereby all Channels installed with :MOnitor will be displayed on the Monitor.

:LINKTime <Value in 10s>    (Valid Values: 6 .. 120)

:LINKtime <Value in 10s> 30 is an advised value.

After this time has run out the Link will be tested by means of an RR-Frame, provided however that during this time no other Data has been conveyed. If there is no return acknowledgement to this RR, the    Connection will be severed after Polling and the run out of RETRY.  This is the case if the out and return time of the Link has deteriorated, or a Digi or the QSO-Partner has switched off without severing the Connection in the normal manner.

:MAxframe <Value>     (Valid Values: 1..7, recommended Value: 3 (For normal usage QRG))

Is the maximum number of Frames, which will be sent in a sequence before an Acknowledgement will be awaited. The values from 1 .. 7 are permissible, other settings are able to be utilised according to your own testing.

:MYcall <CALL1> <CALL2> ....

With the :MYcall-COMMAND the Station Call Signs previously set in SCC.INI are able to be altered. (But not permanently. Only in SCC.PAR until the next run of PARA.) Contrary to Digicom practice, here the Incoming Connect Requests will be allocated one of the indicated Call Signs in the List.  If it should be required that an outgoing Connection is required that is not the first of the MYcalls, the Command :Xconnect should be employed. An alteration of MYcall is also possible during the course of a live Connection, the affected Port retains it's old MYcall up to the Disconnect.

:Paclen <Value>   (Valid Values: 1..256, Default 256)

The maximum Framelength which the Terminal will output. The Frames that the Station outputs as a Digipeater, will be the same as the Original Length that the Station using the Digipeater facility is using. It is important that a value of 1 (unreasonable) up to 256 is used, with the use of 0 it will Crash. Installation of Values above 256 can give amusing effects. The Value used should be dependent upon the Quality of the Connection to the Opposite Station. If you have good Retry free Connection, then 256 is able to be installed, if it is bad then 128 or 80 ought to be used. In any case especially relevant for Paclen is that during the transfer of Files with individually issued lines, nothing over 80 Characters per Frame is issued or is received.

:Quit

Behaves as Disconnect and severs the Connection, but only if all Data awaiting transmission has been sent and acknowledged, or the Quittimer (see :TQuit) has run out. Recommended that it's principle use be for    the severance of a Connection by Remote Control.

:RESptime <Value in 100ms>     (Valid Values 1..10, Default 1)

Gives the time after which the Partner's Packets will be Acknowledged. This value must always be different to 0, but not so large that the Partner will already have polled again (Consequently smaller than his   Frack). With larger values on both sides Acknowledgement Packets (RR) will be able to be reduced, if :RESptime has a Value that is very much greater than :DWait.

:RETry <Value>      (Valid Values: 1..100, Default: 20)

Gives the maximum Number of Packet repetitions or of a Poll, before the dismantling of the Connection will be commenced. Valid also for the Number of the Connect Requests with which to build a Connection. With Connect and Disconnect only RETRY/4 - attempts will be undertaken. Via a Digipeater on grounds of the unknown Number  of Autorouting-Digis (Inc NODES):  RETRY*2 attempts will be made.     (You are recommended to test this for yourself, as the Translators Copy does not agree with /4  and *2 [It is /2 and *1 respectively])

:TAil <Time>    (Valid Values: 0..100 Recommended: 1)

Time in 10ms-Units, after the Sending of a Frame to the closing down of the TX (PTT OFF) (That is to say a TXDelay at the End of the Frame).  This is important for Full Duplex Operation and for Modems with "longer conduction" (AM7911, should be set to 3). If this is not be observed for such a Modem, the TX will be switched before the Stop Flag has been sent, if this case arises the Opposite Station will not be able to decode the Packets !! Tail is able to be installed for each Channel in Multi Channel Operation, it requires the Channel Number before the Value.

:TInfo <Value in 100ms>    (Valid Values 1..100, recommended Value: 4)

is the so called Info-Timer, therefore the Time that will be awaited for the filling of the TX Buffer by the eventual Data-End Flag before the Data will be transmitted. This Time delays for example, that with Presses of two RETURNS one after the other, each Character will be packed into a Frame and will be sent. Default value 4, is preferable but can also be selected by experimentation.

:TQuit <Value in 10s)    (Valid Values 1..60, recommended Value: 30)

is the Quittimer. This Timer severs the Connection after the issue of :Quit, if Data still in hand. Purpose of this Timer is to sever the Connection if because of a Bad Link, the data is unable to be sent.

:TXDelay <Time>  (Valid Values: 1..40, Recommended Value: 12..28, dependent upon TX/RX Char).

This is the Time between the PTT being opened and the sending of the valid Data. The time is given in 10ms-Units. With the employment of more Radio Channels, e. g. with the SCC-Expansion Card the Channel    Number must be inserted before the Time, for which it is valid, therefore if :TX 0 20 is set, the TX-Delay for Channel 0 will be 200ms. With TXDelay it is able to compensate for a slow opening Squelch of the Partner, as well as the sending station's PTT, A Value that is too high increases therefore the QRM unnecessarily. In generala Value between 16 and 24 is reasonable. Warning: The newest Software Version for the BayCom-Digis contains a TXDelay-Monitor. If a Station arrives with 30 or more audible Flags (TXDelay therefore much too high) Packets from this Station will not be accepted. We request that this be considered when installing TXDelay.

:Xconnect <Sender> <Endcall> <Digi1> <Digi2> ...

The Command Xconnect has a Special significance. It allows an Outgoing Call under a different Call Sign, without altering the Station Call permanently. If there is a requirement for a Special Connect to be made with another SSID for a one off occasion, or for a visiting Amateur's use, this is the Command to be used. It does not change any Calls in SCC.PAR on exit from the Program for continued use by the Station.

Example:    :X DL8MBT DB0RGB DB0AAB

This will seek to build a Connection under the Call Sign DL8MBT, to DB0RGB via DB0AAB.  At the End of the Connection the Port returns to the old MYCALL.

3.4  DISPLAY OF THE INFORMATION ON THE SCREEN

Generally before the First Start of the Program, the Screen-Video-Card Adapter Parameter in SCC.INI must be installed. If however after the Start of SCC.EXE a strange or unintelligible Display becomes visible,  please read further in the Chapter ERROR CORRECTION. The Command VA (Video Adapter) in SCC.INI is set at 0, so that by automatic recognition if an EGA-Card is in use it will be the Display will be switched into a 43-Line Display. With a VGA-Card the 50-Line Mode will be employed, and furthermore, with higher definition Cards 60 Lines. If this is not required (Letters too small ?), VA 2 should be installed in SCC.INI. The Parameter VA is also able to be changed from the Program, but the change will not be effective until after the next  Start of SCC. But such an important alteration should always be made in SCC.INI, otherwise it is possible at a later time to receive a big Surprise !

:ANswer ON/OFF

The Personalised Greeting will be toggled by this Function. If ANswer is ON, each incoming Connection will cause a search to be made in the Directory of the Diskette or H/Disk for a Text File named <call>.CTX    (e.g. DL5RL.CTX) and the Text will then be sent, in agreement with the Call Sign. If there is no corresponding Text available then the Normal CTEXT will be sent. With ANswer OFF no search will take place for this File, which for Portable operation will save power, and from Disk operation gives  consideration to the ears. On the contrary to CText, ANswer-TEXTS can only be sent when the Terminal Program (SCC.EXE) is running, because L2 has no ability to access the Disk.

:BLink ON/OFF

Toggles the Normal Flashing of the L2 Rectangle in the top R.H. Corner of the Screen. The reason for this, is that the Flashing can interfere with other Programs that use this corner of the Screen, or so that the    Boss is not able to see it when Packet-Radio is running in the Background on the same Computer (Please do not laugh, because we had a large number of requests in this direction !!)  This Command only functions if it is installed in the SCC.INI Data.

:CBell ON/OFF

Toggles the Bell which is issued on a Connect - Disconnect or a received CTRL-G, ON or OFF. Important for the night sleeping times, when one does not want his peace disturbed, or in order not to awaken the adjacent Desk occupant in a QRL.

:CLear

With the Command CLear the entire Screen or parts thereof is/are to be cleared.  Valid for the immediate selected Port. If the Terminal is called by SCC -C, then the Screen will be completely cleared at the Start of the Program.

| :CLear | Clears the RX Window |
| --- | --- |
| :CLear 1 | Clears the TX Window |
| :CLear 3 | Clears the Monitor Window |
| :CLear 123 | Clears all Windows |

:COMmand ON/OFF

This Command Toggles whether or not the Line following a Command Line will be opened as a Command Line.  That is whether the next Line following a Command Line will automatically commence with a Colon at the beginning of the Line or not. YES set with ON or NO set with OFF.

:CText <TextTextText>

Using :CText, up to a Line of Text will be sent automatically to the Opposite Station on Connection, short Information about QTH and the System respectively.  Availability should be included. The CText will    be sent even if the Terminal SCC is not running, but only the resident L2. For that reason it is reduced to one Line, so that the minimum possible RAM space will be occupied.

:ECho ON/OFF

With this Command it can be decided whether the Characters typed in the TX Window are copied to the RX Window. These Characters will be shown in the same Colours as in the TX Window. This enables the Tx and RX Text to be able to be distinguished one from the other.  With Remote-Operation this Function has another meaning:    //e ...text...    sends the accepted Text circulated back to the 'owner' Station. This    is especially to test the aforesaid Connection's running time.   WARNING: With Remote Control only //e (Lower Case) is able to be sent, it may NOT be sent as //Echo !

:ENglish ON/OFF

This Command Toggles the Program Information (Responses, Error Messages etc.) between the German and English Languages. The Program PARA.EXE, L2.EXE and OFF.COM are not changed by this Command from German, but these Outputs are relatively meaningless and therefore on this account for non-German speaking Users irrelevant.    (Trans. Note. THE PROGRAMS DO CONTAIN AND SHOW A COPYRIGHT NOTICE IN GERMAN. SO PLEASE DO NOT BREACH THIS COPYRIGHT.)

:German

In order that the Champions of German Characters as well as the Friends of IBM-Umlauts can please themselves in PR-Text, the conventional German Umlaut Display is able to be selected with GERMAN ON, and with GERMAN OFF the IBM-UMLAUT. This affects only the transfer and Display of the Frame, the Keyboard remains undisturbed by this Command. With German ON { } and [ ] cannot be transferred, | and \ will not be displayed.  With German OFF all Characters remain in the original.   (IBM-Character Set). Also the Installation will be taken into account by the Commands WRITE and READ (see these in their section).

During the course of a live Connection (and then only) the Key Combination ALT-U can be used to change between DIN and IBM-Character Set. (U in the Status Line shows that German ON is operative) As soon as a Character is received, that indicates that the QSO-Partner is sending and receiving IBM-Characters the U Status Line will Flash, to enable the incompatibility to be indicated, it should then be Toggled with ALT-U. ALT-U is only able to toggle during a Connection, and the setting will remain for the duration of this Connection only.

:KNax ON/OFF

With this Command a Knack-Sound that is heard with each Line received at the Terminal, is able be Toggled ON or OFF. In Version 1.20 the Knacks were permanently active (but not mentioned in the Manual), this led to a string of Messages to us: "My Computer, with BayCom gives out such Comical Sounds from itself..."  This attribute is now switchable.

:MCalls <Call1> <Call2>

Set for the acceptance of the desired Monitor Operation, to be selective with incoming Call Signs. See :MSELECT

:MHEADer ON/OFF

Toggles the Output of the Address Field in the Monitor Screen onto a separate Line (ON), i.e. Between the Address Field and the Info Field a Carriage Return, or on the same line (OFF) Installation is to your choice.

:MHeard

Shows all the Stations that have been heard in the previous Minutes (Without Time and Path). It is variable according to the following:

| MH * | All Call Signs in the last 120 min, without Path. |
| --- | --- |
| MH # (Hash) | All Entries in the last 15 Minutes, with Path. |
| MH <Call> | Time, Path and SSIDs, if the Call was heard. |

MH <Call>: Instead of the complete Call Sign, it is possible to give part of a  Call Sign (e.g. MH DG shows all Stations that were heard, in which part of the Call Sign has DG present :- DG3RBU, DL5RDG etc.) With a Multi Channel System it s able with the issue of a P<Number> to select an individual Channel (e.g. MH P2 *), P0 is the first Channel) If a Time is given, so follows the Output of the "since this time"    Stations Heard (Example: MH * 20 = all Entries of the last 20 minutes). All Times will be given in the Format HH:MM whereby no Absolute-Time will be given, but the Time that has passed since the last start up of the Station.

:MOnitor <Channel Number>    (Default Value: 0)

Determines the Modem Channel, from which the Monitor will Display. :MO 0123456789 All channels monitored, MO 0 for example displays only the first Channel. This Command is only valid for a Multi Channel Operation with a SCC-Card, if for example with the use of a Serial Modem a Value of 5 is set (or any other), the Monitor will be completely switched off.

:MSelect <Value>    (Installation ranges 0..3, 11..13, others will install 0)

Allows certain Packets to and from  Special Stations to be selected so that they are either displayed enhanced or excluded. Determined QSOs are able to be more easily followed. The following Values are able to be set:

| 0 | Monitor-Selection off, all arriving Frames will be shown. :MOnitor must naturally be set for the Channel concerned. |
| --- | --- |
| 1 | QSO-With Cut. Here those Call Signs entered under MCALLS connected in pairs (and only when they are constantly Connected) and when BOTH Call Signs are to be found, will be displayed on the Monitor. It is also possible to display more than one pair at the same time. |
| 2 | MFROM-Usage. Only Packets FROM the Call Signs in the MCALLS-List are able to be displayed on the Monitor. |
| 3 | MTO-Usage. Only Packets TO the MCALLS Call Signs will be displayed. |

If the Value 10 is added (11, 12, 13) the Function will be inverted, i.e. the Selected Call Signs will NOT be displayed, otherwise the action is completely normal. If no SSID is given, Calls under all SSIDs are valid.

:QText <....text....text...>

This Text will be sent out by the issue of :Quit at the Host Terminal or by //Quit from the Partner.  There is some disagreement with the usage of this idea and constant requests for this to be deleted.

:Sattrib

The Colour Installations of the Screen are prepared by the Installation of the Parameter Sattrib. These Installations are naturally able to be altered "Live" from the Screen. They are actioned by the Command:

:Sattrib <Field Number> <Colour Combination>  (The numbers valid for the Field are the Numerals 0-9)

They have the following significance:

| 0 | TX Window |  |
| --- | --- | --- |
| 1 | Upper Status Line |  |
| 2 | STOP-Indicator | in the upper Status line, created by the Halt Output Command given by Shift-Tab Keys. |
| 3 | RX Window |  |
| 4 | Lower Status Line | Indicator of Connect-Call-Sign and Port No. |
| 5 | Lower Status Line | Indicator for non selected and not connected Ports. |
| 6 | Lower Status Line | Indicator for the immediate (selected) Port on the Screen |
| 7 | Monitor | Address field of the Frames |
| 8 | Monitor | Info-Field of the Frames |
| 9 | Text Colour | In the Help-Window |
| 10 | Help Window | Colour of the selectable items |
| 11 | Help Window | Colour of the selected items |
| 12 | Control Characters | therefore CTRL-Z etc. Must be different to the Colour-Combinations of the Fields 0, 3, 7 and 8 otherwise it will be invisible |

For the Colour combinations, a two position Hexadecimal number must be used, separated for Background and Foreground. (Foreground is the Character itself, Background is the Colour on which it stands. For the Colours

| 0 = Black | 8 = Dark-Grey |
| --- | --- |
| 1 = Blue | 9 = Light-Blue |
| 2 = Green | A = Light-Green |
| 3 = Cyan (turquoise) | B = Light-Cyan |
| 4 = Red | C = Light-Red |
| 5 = Magenta (violet) | D = Light-Magenta |
| 6 = Brown | E = Yellow |
| 7 = White | F = Light-White |

The Numbers 8-F are only valid for the Foreground. As Background Colours the adjacent left hand Number from 0-7 produces Flashing. To make it more simple to understand, here are a few Examples:

:S 8 04

The Displayed Info-Field in the Monitor will be Red on a Black Background.

:S 2 C7

The Stop-Indicator (From Shift-Tab) White on a Flashing Red Background

WARNING: Alterations in the Status Lines operate directly after the appropriate Command. Alterations for the individual Windows are valid for the first time with appearance of new Text and not for the existing    displayed Screen Contents! (Important for the areas 0, 3, 7, 8)

The choice of Colours for the End Effect, remains primarily one of personal taste and the Screen Possibilities are a free choice. Similar Combinations (e.g. 00: Black Characters on Black Background) makes    in itself generally not so good a choice as those with plenty of Contrast, HI.

:SAVeports <Number>

gives the Number of the Screens that will be saved to the Disk at the ending of the Program (ALT-X). With slow Computers it is recommended that only one Port is saved or completely switched off :SAV 0. Only the Parts visible on the Screen will be saved on the Disk at the closure, the Scrollable Text will not be secured.

3.5 COMMANDS FOR THE SAVING AND TRANSMISSION OF FILES.

As is possible with all Terminals, BayCom Files can also be read from a Data Support System and also able be saved. But it is to be noted that BayCom is unquestionably not in itself THE Program for Mailbox-Milking-and-Storing-Fans. It can be stated absolutely, that by the use of an unfavourable Equipment Combination (Slow Floppy) it can lead eventually to the infrequent non decoding of Frames, one Frame that will, therefore be received correctly, will suddenly not be decoded, this is because the Floppy- and PR-Interrupt arrive a little too close together. This does not mean that it creates Errors in the File, the Frame must await a re-send from the Partner. (Put shortly it retries!) But that really, is seldom the Case. The following File Commands are provided for File Management:

:Edit <Filename>

The Edit-Command opens a file on the Diskette or H/Disk and saves everything in it that is Output from the Keyboard and ended with the RETURN-Key. The text is entered as if it is to be sent, but in reality it will not be sent, but written exclusively to the Disk. The Editing is able to be ended by :Edit OFF (AUS) The Command is not to be thought of as a large Editor, but should make possible, the saving of sudden occurring "flashes of inspiration" without having to leave the Program or to "Overwrite" by RETURN to the Storage Medium. If a File exists with the same Name it will not be deleted, but the new Lines added to the end of that file.

:Read <Filename>

Sends the specified Text File to the Opposite Station. (That is to say the opposite of the :Write-Command). Example :R C:\BAYCOM\SCC.INI. This will Read and send to the Opposite Station the Text File, SCC.INI which is in the BAYCOM Directory on the C: Drive. In addition if :German ON is installed, the German Umlaut in the DIN-Character Set will be changed, this must be carefully watched.

:RPrg <Filename>

As for :Read, but for binary Files. The specification of the Post Office relative to File Transfers is to be unconditionally observed !

(TRANS. NOTE This is not a YAPP Binary File Transfer, and as yet I have been unable to test Transfers either between BayCom Users or other Systems and BayCom. Please send details of your experiences with this to G0KIU for inclusion in Manual Updates.)

:View <Filename>

With the Command VIEW, a Text File is displayed in the RX Window. With short Files the Contents are able to be read by the use of Scrollback, for longer Files it is recommended to use the STOP method, using    SHIFT-TAB-Keys or by the + on the Number Pad. Also it is possible to use the full DOS-Path specification.

Example:    :V C:\BAYCOM\SCC.INI

:Write <Filename>

With this Command a Text File will be opened and the entire incoming Text from the Opposite Station after the issue of the Command, will be saved to Disk. The Filename may be any chosen name up to 8 Characters plus DOS-Extension and also contain a complete Path. To end the saving of the Text :Write OFF

Example:    :W C:\PR\SCC\INFO\BAYCOM.TST

:W TESTFILE (The File by the name of TESTFILE will be saved in the Directory from which BayCom was started). At the same time the usual Packet-Radio Carriage-Return (CR) which under DOS gives a customary CR-LF will be converted. With :GERMAN ON the German Umlaut will be converted from the    DIN-Character Set into the IBM-Character Set.  In order to Print incoming Data on a Printer the WRITE-Command should be used. With :WRITE PRN the Printer will be switched on line. Files will be managed as under MS-DOS Commands, it is in this Mode that principally every instruction is available (e.g. :W LPT1, :W COM2 etc.) A single Command for the Printer Usage does not exist, in order to give the largest possible flexibility of choice.

In this connection it is able to point towards the indescribable advantageous capabilities of the NUL-Device under DOS. Which for the security of irretrievable unimportant MailBox Contents, outstanding Service is able to be performed and naturally with :Write NUL able to be addressed. The Write-Command is able to be integrated in SCC.INI, in the Model-INI there are already four Files prepared for the Writing of incoming Data.  (To activate clear the ; at the beginning of the Line(s).

:WPrg <Filename>

With WPrg a Binary File will be opened, that is; all incoming Characters after the issue of the Command will be saved to the Disk, each one without any modification. Similar to :Write, to end the saving to Disk by use of :WPrg OFF (AUS). Auto Binary Transfer as for example with Turbopacket, at the moment is not available with BayCom. The Remote Control is blocked during the course of Receiving Binary Data.   Functioning as a single Command  //WPrg  OFF (must be entered thus) To be observed is :- that a Carriage Return (CR) must come before this Command is used, which is not normally the Case at the end of the Binary Data.  (Before using the //WPrg OFF a return should therefore be sent.)

3.6  LOG BOOK MANAGEMENT

As a further new offering, we are pleased that in this Version of the Program there is the provision of a Log Book Facility (Positively only when the Terminal SCC is running, the resident L2 unfortunately is    unable to write to the Disk !!).  Two Commands for this Function are relevant:

:LOG ON/OFF

This Command determines whether or not a LOG Book is maintained. The entries will be written to the File SCC.LOG, and are able to be read or edited with a favourite Text Editor. New Entries are always added to the end of the File.

:EXitlog ON/OFF

This Command determines whether the LOG ON writes immediately to the Disk (:EXitlog OFF) or collects it in the RAM and then write it at the Exit from the Terminal (ALT-X).  Normally a permanent Log Book Control does not disturb the Computer, but it is able to be troublesome with Portable Usage (Higher current requirement, through the running of the Disk or Floppy). The disadvantage of the RAM-Intermediate Storage is however, the seizing of Log Working (RAM) Space and that with a Hard Reset (Computer Switch off or Crash) the latest Log-Data will be lost. Entries can also become corrupted by the overstepping of the Data.  Barrier in the RAM and then False Data will be written.

The Log will be laid out in the following Form:

14.05.1991 20:28 - 20:34 P0 Connect    DG3RBU      > DB0EV

14.05.1991 20:32 - 20:32 P0 Failure    DG3RBU      > DK5RQ

P0 means from Channel 0 (only relevant with Multi Channel Usage), Also the Sender's Call Sign will always be logged against the Target Call Sign . (e.g. for use with XCONNECT).

3.7  Other Commands:

:DOsclear ON/OFF

This Command is only available when contained in SCC.INI!! This Command decides whether before the start of the Terminal the existing Screen will be saved (OFF) and then at the end of the program restored; or not saved (ON)

:Info

This Command is to send (mainly for the Remote application) the Info text to the Opposite Station. This Text will be created with an Editor or with the :EDIT-Function of the Program and must be stored under the Name INFO.SCC in the Current Directory on the Floppy/Disk.

:INSMODE ON/OFF

This Command determines, whether the Keyboard operates in the Insert  Mode (ON: a Key that is pressed moves the Character under the Cursor one space to the right and then inserts it's Character in the vacant space.) or in the Overwrite Mode (OFF: The Key that is pressed replaces the existing Character at the Cursor with it's own). This Command exists only in the SCC.INI File, however it is able to be toggled by    the use of the INS Key.

:J

With use of pw the Pass Word Algorithm for BayCom-Node and DXL-Mailbox will decoded. After the Request the Program sends out on the corresponding Port "pw" and answers the 5 returned numerals accordingly from of the Contents of the Data SCC.PWD. The Terminal is also able to administer a Password, it will have demanded with //J to be given 5 Numbers.Correctly answered with the  corresponding Characters from SCC.PWD, it will open the complete Remote-Possibilities to the Opposite Station.

SCC.PWD answers, if the Opposite Station will completely open the Remote possibilities The Password employed, comes from 80 Characters and is saved in the File SCC.PWD.  To complete, the Parameter Compiler PARA is called and L2 to start. For the application of the Password for TheNet one allows the 5    selected numbers to arrive and then set the :J in front of these 5 Numbers. With the operation of Return the correct letters will be transmitted.

:Oshell

With the Command :Oshell it is able to quickly Shell to DOS, but meanwhile the Terminal and all Connections will continue to run in the Background. With the issuing of :Oshell (or shorter :O) the Command Interpreter of DOS (COMMAND.COM) will be called first and then the Normal DOS-Prompt will appear. When using a Floppy (No H/Disk) it is naturally important to keep the Boot Disk with the Command Interpreter on the Drive. It is now optionally capable of having complete control (Within the limits, that only very little RAM is available !). With the issue of EXIT <RETURN> and followed by any other key the Computer is returned to the Terminal. It is also possible, to attach the required DOS-Command after the :O-Command, for example :o dir B: /w This shows the Contents of the Disk B: in the width Mode. After the issue of the Command one returns to DOS by the pressing of any Key, consequently it must not be given the command EXIT. The :O-Command replaces certain of the Commands contained in Digicom e.g. for the Floppy (:dir, :f etc) and simplifies therefore the operation of the Program. Also Remote Control of the Command is now possible, but misuse is now able to be easily controlled, accordingly one should bar the Commands with the Help of the RCMD-Function. The storage available for the instruction is not very large and not sufficient other than compact applications, this Command is considered chiefly for DOS-Commands such as COPY, DIR etc. Unfortunately as before, with the use of :Oshell-Command in Connection with the RAM management of DR-DOS 5.0 there are some problems that unfortunately up to now we are not able to elucidate. Under DR-DOS it is preferable that one discontinues the issuing of Oshell, because in the last resort one must re-boot the Computer (Report: Insufficient Memory)

:SYstem

ends the Program. The Function corresponds exactly to the ALT-X key  Combination  and is principally considered for the Remote Control Shut down of the Program. This is only sensible when the owner of the    Station wishes it. This Command therefore should not be made freely available by RCMD (see above) but to be accessible only after the Correct Reply to a Password.

:Users or :CStatus

Gives out a List of the immediately Connected Stations. The Format agrees extensively with the handling of the RMNC/FlexNet-Nodes. CStatus is not to be confused with the CSELECT employed in SCC.INI!

:VErsion

Gives in three Lines on the Screen the Current Version of BayCom in use as well as the most important Configuration Data (Insert report)

3.8 REMOTE CONTROL.

In the BayCom-Terminal there is the possibility of Remote Control by the Opposite Station. The requirement to achieve this is the freeing of Remote Control through the issuing of :REMote ON (EIN). Unlike Version 1.20 there exists a RCMD-Command, which may be in SCC.INI or be issued from the Terminal.

Example:    :RCmd CStatus Info MHeard Quit Users

gives the freedom to Remotely Control the Commands following RCmd

:RCmd * makes all Commands freely available. The quantity of the individually explicitly given Commands is limited to the contents of one Line, but it should be possible by employing Customary Command  abbreviations, which are always in CAPITALS in this Manual.

(Example: :RC CS I MH Q U H) to accommodate all required Commands.

It allows for example all Commands to be open for remote Control, if a Password-Data SCC.PWD has been created and then by use of the //J Command the Password Mechanism has been correctly decoded. Detailed information for that purpose.  The Command :Help will not be able to be used by Remote Control.

3.9  UNPROTO-TRANSMISSION

With Digicom, the Output goes out on it's journey on all Ports without a Connected Station as Unproto-(UI)-Transmission. With BayCom such Output without Commentary will be swallowed. If it is required to send an Unproto-Transmission on it's journey, then such text must be entered in the TX Window of the F10-Screen and transmitted by <RETURN> Naturally an Unproto-Address or Digipeater Chain is able to be given. This allows the display to be sent by the issuing of :C DB0AAB DB0RBG, in the TX Window of the F10-Screen, whereby DB0AAB is the end Address and DB0RGB the Digipeater Station via which the Information will be sent. With the issue of this Command from the F10 therefore no Connection will be constructed. (Connections are only  built from the individual Ports), but purely an Unproto Address of the Program attached. The Unproto Address for Beacon Transmission will be installed with :BAdress.  An indicator only for Specialists: The Data in the Unproto-Mode will be sent as UI,P. That means that the Poll-Bit will be set, this has the advantage, that the sending for Test Purposes is able to be very usefully  employed: in accordance with AX.25-Protocol has to react on such a Packet with a DM and is able to be observed on the Monitor.

3.10 SHORT OVERVIEW OF THE COMMANDS AND KEYS

COMMANDS  (Allowed abbreviations shown in Capitals.)

| ANswer ON/OFF | Personal Connect Text on/off toggle |
| --- | --- |
| BAdress <Call> | Address for Beacon |
| BEacon (Number> | Beacon Interval Time |
| Btext <Text> | Beacon Text Contents |
| C <call> [Digi..] | Connection setup, customary usage |
| CBell ON/OFF | Connect Bell/CTRL-G: Bell on/off toggle |
| CLear [No] | Screen Window clearance |
| CNot <CALLS> | Station Calls, which are not able to Connect |
| COMmand ON/OFF | Automatic Colon set or not set after a Command line. |
| CStatus | Outputs User List |
| CText <text> | CTEXT Contents |
| Disconnect | Sever the Connection (disconnect) |
| DCall <call> | Individual Digi Call Sign Installation |
| DISCTime <Number> Timeout | Timer for Inactivity |
| DWait <Number> | Channel-Free-Waiting Time |
| ECho ON/OFF | TX Screen Copied or not to RX Screen |
| Edit <name> | File to be created from the Keyboard |
| ENglish ON/OFF | English/German Program reports toggle |
| EXitlog ON/OFF | Log update at the Terminal End or Immediately |
| F <Number> | Frack Time up to which the Poll waits |
| German ON/OFF | DIN/IBM Character Set Toggle |
| Help | Help Call |
| HBaud <Number> | Baudrate installation |
| I | Info Text Output |
| IPoll <Number> | Maximum Info Poll-Length |
| KNax ON/OFF | Knacks at the receipt of Data Toggle |
| LINKTime <Number> | Link Check, if no data has been conveyed |
| LOG | Log Book ON/OFF Toggle |
| MAxframe <Number> | Maxframe, maximum number of unacknowledged Frames |
| MHeard <option> | MHEARD-List Output (Options * # <call> ) |
| HEADer | Line-Feed in the Monitor between Address/Info Field |
| MCalls <Calls> | Calls to be Specially Monitored and Displayed |
| MOnitor <Chan.No.> | Select the Channel to be Displayed on the Monitor |
| MYcall <call> | Individual Call Sign |
| Oshell <Command> | DOS-Command Shell |
| Paclen <Number> | Maximum Framelength |
| Quit | Disconnect if all has arrived. |
| QText <Text> | Quit Text Installation |
| Read <Name> | Text File Transmit or read if not connected |
| RCmd <Commnads> | Commands allowed for free use by Remote Control |
| REMote ON/OFF | Remote Control ON/OFF Toggle |
| RESptime <Number> | ACK-Delay |
| RETry <Number> | Maximum Repetition attempts to send Data or Connect |
| RPrg <Name> | Binary File Transmit |
| SAttrib <Attribute> | Screen Colour setting |
| SYstem | End the Program |
| TAil <Number> | Number of stop Flags |
| TInfo <Number> | Infosend-Timer |
| TQuit <Number> | Maximum Time after Quit |
| TXDelay <Number> | TX Delay Time |
| Users | Issues the User Lists |
| V <Name> | View a Text File |
| VErsion | Issues the Switch On report |
| Write <Name> | RX Text File to Disk |
| WPrg Name> | RX Binary File to Disk |
| WW <Number> | Position of the Line Feed (Wordwrap) |
| X <my> <call>[<digi> .. ] | Connect under an Optional Mycall |

Keys:

| F1 - F8 | Selection of the individual Ports (according to Number) |
| --- | --- |
| F9 or - on the Keypad | Moves the Cursor to the next Window Below or from Monitor to the TX Window |
| F10 | Monitor and Unproto-Screen |
| ESC or TAB | Switches between the Command and Data Port, sets or clears a : at the Col 1 of the Line |
| + Key on the Key Pad or SHIFT -TAB | Stops the Output from the selected Port |
| CTRL-RETURN | Line Feed, without the Line being actioned (As SHIFT-RETURN in Digicom) |
| PgDn | Text Scrolled one Page Down |
| PgUp | Text Scrolled one page upward |
| CTRL-PgUp | Monitor Window enlarged, Text will be carried with it |
| CTRL-PgDn | Monitor Window reduced, Text will be carried with it |
| CTRL-HOME | TX Window reduced, Text will be carried with it |
| CTRL-END | TX Window enlarged, Text will be carried with it |
| CTRL-F1 | Help Text selected at the Cursor retrieved |
| ALT - X | Program SCC.EXE will be ended |
| ALT - 1 | Set the Cursor in the TX Window |
| ALT - 2 | Set the Cursor in the RX Window |
| ALT - 3 | Set the Cursor in the Monitor Window |
| ALT - U | Toggle the Umlaut |
| ALT - F1 | Repeat the previous Help Text |
| INS | Toggle between Overwrite - and Insert- Mode |

Part III: APPENDICES

1. ERROR CORRECTIONS.

Here naturally, it is only able to suggest remedies for a small part of the possible Errors that you may encounter, it should however help to disclose Errors in Hard- and Software.

If after the Start of L2 no Flashing Rectangle appears in the Top Right Hand Corner of the Screen or the Computer Crashes:

- Check whether the correct Serial Port has been installed. It is only possible to use COM1 and COM2. Is the Port actually available as well as the Interrupt for it correctly jumpered ? (With most Comms.-Cards      it is installed by the means of a Jumper, see Computer-Manual).

- Are other Resident Programs running at the same time (Except the customary DOS Drivers), sometimes collisions can occur. If so, these other Programs should be removed and further tests made without them.      (NOTE OFF.COM only Functions to remove L2 and not other Resident Progs.!!) Then if there is no Flash visible, be on the look out for the worst...presumably BayCom is not usable on your Computer, for       which many reasons are known.

The following Computers are known to give Problems:

- NEC-Multispeed

- Atari-Portfolio (an entirely hopeless Case)

- Certain Sharp-Laptops.

- Bondwell B200 under DR-DOS

BayCom should run in each case on the following Computers:

- all "large" PCs without exotic Communications Cards/Ports (if an 8250 is on the Card, in any case)

- The following Laptops have already been tested:

- Epson PC-Portable

- Olivetti M15

- Bondwell  B200, MS-DOS (Certain Current supplies of the Modem to be modified)

- Dell 386

- Sundry Toshiba-Types

A CONNECT REQUEST IS STARTED BY :C. ON THE MONITOR SCREEN NO CONNECT    REQUEST IS OBSERVED.

- If the rig is also not Transmitting, then presumably L2 is not functioning! End SCC with ALT-X and observe, whether the Flashing Rectangle in the upper R.H. Corner disappears.

- Is the Squelch remaining open ?

PACKETS WILL NOT BE DECODED.

- Prove, that L2 is running (Flashing Rectangle).

- Prove, that the :MONITOR-Command of the required RX Channel is set correctly !

- Check the RX sections of the Modem for Broken Tracks or dry joints.

- Prove, that the Correct Signal Level on Pin 4 of the TCM3105 is present. (it should be >100mV at this time).

- With an Open Squelch, continuous Data should arrive. Follow this with an Oscilloscope to the CTS-Pin of the Port. In the case that there is no contiguous Data (+5V-0V-Rectangle) you should now switch off and      once again check the PCB for Components, Solder bridges and broken tracks! Also: Is the Crystal of the TCM3105 oscillating ?

- Data arrives regularly (it shows in the Program Status Line RECV with an Open Squelch), inspect your RX-Signal. On the Oscilloscope, with PR-Signals arriving two parts should be able to be recognised      (1200 and 2200 Hz, the two PR-Signal Tones). The Amplitude of these must unquestionably be the same. If they are not, a Low- or Highpass (simple RC Filter) is required in the Signal Path.

L2 FUNCTIONS, AFTER THE START OF SCC NO TERMINAL INTERFACE SHOWS.

- In correct Video Adapter installed in SCC.INI.

CURIOUS SCREEN DISPLAY COLOURS

- Colour Combination SATTRIB installation or VA in SCC.INI altered. In the Second Case re-start PARA and through it begin again with the Default Combinations from the Sample SCC.INI.

ALTHOUGH A PARAMETER HAS BEEN ALTERED, THE CONTENTS OF THE PROGRAM HAS    NOT APPEARED AS EXPECTED.

- Many of the Parameters will only be utilised with the Initialisation Routine. It is necessary therefore, after the alteration of the Parameters, to end L2 by means of OFF and a restart commenced.

NOTE: Important  Alterations to the Parameters SHOULD NOT be made from the User Interface but always in SCC.INI. Alterations of SATTRIB in the Window Fields are only observed with newly arriving lines, not those already on the Screen.

ERROR REPORTS FROM THE RUNNING OF THE PARAMETER COMPILER PARA.EXE    (e.g. WITH THE START OF L2 AFTER THE ALTERATION OF SCC.INI)

- The Format of SCC.INI has been damaged. One should compare the Line in question with the supplied original-SCC.INI. Perhaps a ; before a Commentary has been erased or a false Parameter typed (e.g. Text      instead of a Number).

BELL SOUNDS ONCE, CURIOUSLY.

- In order for the Bell to be created, BayCom at the Start of the Program must obtain the Speed of the Computer. Then during the course of the operation the Frequency Value is reduced or increased, it alters of it's own accord the Tone of the "Tootling".  Return switching to the old Value or through a Re-Start of L2 and SCC this will be rectified.

2. OVERVIEW OF THE AVAILABLE HARDWARE

No TNC is necessary for the Terminal Program BayCom, but instead a small Modem on the Serial Port. The Conditions to be observed are as follows:

| Signal | 25pin | 9pin | Description |
| --- | --- | --- | --- |
| DTR | 20 | 4 | Send Data +/- 10V |
| RTS | 4 | 7 | PTT, High-active, -10V=RX, +10V=TX |
| CTS | 5 | 8 | Receive Data |
| GND | 7 | 5 | Ground |

As mentioned in the Introduction, the Operation as well as with a Modem on the Serial Port, is also able to be used with an Expansion Card. From the BayCom Authors the following have been developed up to now and they are available as Kits from DK5RQ. partially as Complete Units to Order.

MODEM FOR THE SERIAL PORTS WITH TCM3105

On a small PCB, which requires no external Power Supplies, all Components for a Complete TCM3105-Modem. From the middle of June 1991 a ready made PCB will be delivered, which also has the possibility to be used with digital Squelch.

SMT-MODEM WITH TCM3105

Our Smallest, a complete Modem with TCM3105, contained on two small PCBs with SMT-Components, allows it to be installed inside a 9 Pin or a 25 pin Port Plug Shell !!!!!

MULTI-CHANNEL-EXPANSION CARD (From Autumn 1991)

After many enquiries we have struggled to achieve for that purpose, a four Channel Expansion Card for Users. Equipment: Two Ports with TCM3105-Modems, two ports with Modem Disconnect-Headers for 300 - 9600 Bd.

3.  BAYCOM - USER - AND - USAGE AUTHORISATION

The Program System BayCom is freely permitted to be used in Amateur Radio and may together with the Primer be copied. However we draw attention to the following, the Copyright of the Circuits, PCBs,    Software and the Manual(s) remain with the Authors. A Commercial Employment such as a Sale of Software Copies, Circuits and Manuals is not allowed without our permission ! It has been for us, always    unpleasant at the Fleamarkets to see many OM selling stocks of Digicom  - without having had to do the necessary Work to only line their own pockets.

All Money received in connection with BayCom after covering the Costs will be employed for the Bavarian Digipeater Network, to come therefore for the Benefit of All. However we would rejoice, if you have obtained a Copy of the Program from anywhere and are pleased with it, eventually to send a Small Donation transfer to our BayCom-Konto  This will help to secure the promotion of important substantial Future Developments of the Program.

(TRANS.COMMENT) I have visited the Authors and can only say that the amount of work and their own money that has been used in their projects is considerable.  Although they say that money will be used for The Bavarian Network this is effectively a Test Bed for Amateurs to benefit everywhere and includes the development of interesting things to come.

The Following Kits are now available for ordering:

Program with Instructions (State Disk Format)      20.00 DM

PCB only with Construction Manual                  12.00 DM

Complete Kit. incl. TCM3105                        89.00 DM

Update Kit (Digital Squelch)                       10.00 DM

SURFACE MOUNT MODEM KIT                            94.00 DM

Order by transfer to the under named Konto. Please state what is required. !

Konto of the BayCom-Team:

Rudi Dussmann

Kto 190786-859 at the Postgiro Office Nuernberg, BLZ 76010085

Please write clearly and do not forget your Address !!!

The Paper MODEMTECHNIK FOR THE RADIO AMATEUR(30p.) is available against a DIN A4 Envelope and a contribution for the Printing Costs from DG3RBU

AUTHOR'S ADDRESSES:

Please address all questions to DK5RQ or DG3RBU ! (G0KIU will also answer or obtain answers for UK Amateurs if they so wish.)

Rudi Dussmann, DK5RQ            Johannes Kneip, DG3RBU

Otto-Hahn-Str. 9                Tassiloweg 3

D-8400 Regensburg               D-8400 Regensburg

Denis Godfrey, G0KIU @ GB7BAD or QTHR.

Much joy with the Usage of BayCom.... and if nothing happens at the first attempt, do not up-skittle everything, but first of all a little patience and probing, PR is only the playing field of Radio Amateurs !

(TRANS.COMMENT)  As with most things: When all else fails read the instructions. !!!

3. FUTURE DEVELOPEMENTS

As already indicated, we were surprised at the huge response to the Program, we hope that with this new Version the wishes of many more Users will be fulfilled. Naturally the development will not stand still. Adjacent to the Part 2 suggested Hardware development which is to be included, Software developments are to be further improved.  We are pleased about each stimulation !  However, next to the Terminal also the Digipeater Version to be further developed, Progress but with reliability will not be so speedily offered, as was usual in the last years of Digicom. In spite of that we hope, that with the Version 1.40 to have produced an immediately, universally useful Program, that the comparison with Programs using a TNC has no longer any reason about which to be shy.